# Civil Rights Review – Building Accessibility

[Fill In] School District

[Fill In] High School

[ Please Fill In The Exact Date ], 2026

Facilities Packet Completed By

Name:

Title:

Date(s) of Completion:

Before signing this document, verify that the content you are signing is correct. Please sign below:

Facilities Accessibility Self-Assessment

A review of each facility where CTE classes are offered will be made. The accessibility review will include parking areas, walkways/hallways, ramps, building entrances, libraries, cafeterias, restrooms, locker rooms as well as CTE program areas/classrooms and other assembly areas as determined.

*If every building does not have an accessible restroom and/or water fountain, an accessible restroom and/or water fountain must be within near proximity, and of reasonable distance/ travel, from the non-accessible restroom and/or water fountain.

All agencies are responsible for compliance with Section 504. Applicable accessibility standards are determined by the date the facility was constructed or last renovated, as follows (ANSI, UFAS, and 2010 ADA Standards). All non-compliant areas, irrespective of built /alteration date, must be corrected in accordance with the 2010 ADA Standards.

Accessibility Standards

- Existing Facility under 504 - Built or altered beginning June 3, 1977, or earlier (34 CFR Section 104.22(a))

- American National Standards Institute (ANSI) - Built or altered between June 4, 1977, and January 17, 1991 (34 CFR Section 104.23(a))

- Uniform Federal Accessibility Standards (UFAS) - Built or altered between January 18, 1991, and January 26, 1992 (28 CFR Section 35.151.; (34 CFR Section 104.23(c)))

- 1991 Americans with Disabilities Act (ADA) - Constructed or altered on or after January 27, 1992 and before September 15, 2010 (28 CFR Section 36, Appendix D; (34 CFR Section 104.23)

- 2010 Americans with Disabilities Act (ADA) - Constructed on or after March 15, 2012 (28 CFR Section 35; (34 CFR Section 104.23)

The included checklist provides the framework for the evaluation of accessibility at the school site as described by Americans with Disabilities Act and guidelines published by the U.S. Department of Education, Office for Civil Rights.

Table of Contents

Tips for Measuring	5

Accessible Parking Spaces	8

Accessible Routes / Walkways	10

Stairways and Steps	11

Ramps	12

Curb Ramps	15

Entrances, Doors, and Gates	18

Rooms & Offices (Assembly Areas Available to Students)	20

Elevator(s) Building	22

Lifts	24

Accessible Drinking Fountains	25

Water Coolers & Bottle Fillers	27

Library	31

CTE Classroom(s)	33

Labs/Shops	35

Gymnasium/Auditorium/Weight Room/Other	37

Stadium/Field	40

Dressing, Fitting and Locker Rooms	41

Restrooms (Male/Female)	45

Public Telephones	48

Glossary of Terms	49

# Tips for Measuring:

Parking spaces:

Access aisles: Measure in inches

Parking spaces: Measure in inches

Curb Ramps: Measure in inches

Cross Slope of curb ramps: Measure in degrees or percentages or proportions

Door(s) at Entrance(s) and Exits(s):

Height of door handle: Measure Ground floor to handle in inches

Width of door(s): Measure in inches

Door opening force: The force for pushing or pulling open a door or gate; Measure in pounds

Threshold at doorway: Measure in inches

Signage:

Height if accessible signage: Measure from the floor to the centerline of sign, in inches

Height of characters of signage: Measure in inches

Elevator(s):

Height of call button: Measure from floor to center of call button panel

Width of Door: Measure in inches

Depth and Length of elevator: Measure in inches

Control height: Measure from floor to middle of call button panel, in inches

Height if Emergency Communication: Measure from floor to bottom of panel, in inches

Lift(s):

Height of controls: Measure from floor to middle of control panel, in inches

Benches: Measure from floor to the top of the bench

Toilet/Restrooms

Measure:

Height of grab bars: from floor to center line of grab bars

Height of toilet seat: from floor to toilet seat edge

Height of flush control(s): from floor to control(s)

Height of mirror: from floor to reflecting edge of mirror

Height of toilet paper dispenser: from floor to operable part of dispenser(s)

Height of soap dispenser: from floor to operable part of dispenser(s)

Height of paper towel dispenser: from floor to operable part of dispenser(s)

Height of automatic hand dryer(s): from floor to operable part of dispenser(s)

Height of disposal unit(s): from floor to top of unit(s)Height of lavatory: From floor to rim

Drinking Fountains

Measure:

Height of fountain: floor to spout outlet in inches

Height of spout: floor to tip of water spout

Height of flow: turn flow on, measure from spout to top of arc, in inches

Handrails: Height: Measure vertically from ground to top of handrail, in inches

Tables and counters:

Measure:

Tops: From floor to the table or counter top, in inches

Knee clearance (under tables, telephones), in inches:

From

- floor to lower edge of table.

- depth from table edge to leg of table

- width of work surface

Sales and Services counters require additional measurements than the reception counter tops:

- forward reach approach

- parallel approach

Curb Ramps:

Only measure the width of the ramp section of the curb ramp (labeled “ramp” on the image below). The ramp section of a curb ramp is also known as the “ramp run.” If the curb ramp has flared sides, which can also be seen in the illustration to the bottom, do not include them in the measurement. The ramp run must be at least 36 inches wide.

Running Slope:

The running slope of the curb ramp is the slope in the direction that people travel when going up or down the ramp run. The arrow in the illustration below, aligned parallel to the ramp run and perpendicular to the curb, shows where to measure the running slope.

Cross Slope:

The cross slope of the curb ramp is perpendicular to the running slope. Unlike the running slope, which runs along the ramp, the cross slope is measured across the ramp. The arrow in the illustration to the right, aligned perpendicular to the ramp run and parallel to the curb, shows where to measure the cross slope. The cross slope of a curb ramp, or any accessible route, may not exceed 2 percent.

Gutter Slope:

The gutter is the part of the street that borders the curb. To measure the gutter slope, place the level in the same position as the arrow in the illustration, with one end where the gutter meets the ramp and the other end towards the street. The gutter slope is parallel to the ramp and perpendicular to the curb.

Measurement: Slope is the measure of how steep a straight line is. For a ramp the maximum running slope allowed is 1:12. That means for every inch of height change there should be at least 12 inches of ramp run (length).

Tools Needed: Smart Level or Measuring Tape

Standards Guide, Key Measurement Thresholds

Quick reference of key measurement thresholds. These same notations also appear beside each measurement field in the sections that follow.

| ELEMENT | ANSI | UFAS | 1991 ADA | 2010 ADA |
| --- | --- | --- | --- | --- |
| Door clear width (min) | 32" | 32" | 32" | 32" |
| Door opening force, interior (max) | 8.5 lbs | 5 lbs | 5 lbs | 5 lbs |
| Ramp max running slope | 1:12 | 1:12 | 1:12 | 1:12 |
| Handrail height | 30"-34" | 30"-34" | 34"-38" | 34"-38" |
| Accessible parking width (min) | 96" | 96" | 96" | 96" |
| Van accessible, space + aisle | 96"+96" aisle | 96"+96" aisle | 96"+96" aisle | 132" OR 96"+60" aisle |
| Van vertical clearance (min) | 98" | 98" | 98" | 98" |
| Counter / table height (max) | 34" | 34" | 36" | 36" |
| Forward reach (max) | 48" | 48" | 48" | 48" |
| Side reach (max) | 54" | 54" | 54" | 48" |
| Accessible route width (min) | 36" | 36" | 36" | 36" |
| Overhead clearance (min) | 80" | 80" | 80" | 80" |
| Turning space (min) | 60" dia. | 60" dia. | 60" dia. | 60" dia. or T-turn |
| Grab bar height, toilet | 33"-36" | 33"-36" | 33"-36" | 33"-36" |
| Toilet seat height | 17"-19" | 17"-19" | 17"-19" | 17"-19" |
| Drinking fountain spout (max) | 36" | 36" | 36" | 36" |
| Lavatory rim (max) | 34" | 34" | 34" | 34" |
| Mirror bottom edge (max) | 40" | 40" | 40" | 40" |

Accessible Parking

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, ACCESSIBLE PARKING |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, ACCESSIBLE PARKING |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

SPACE COUNTS

| Total number of parking spaces | Click to enter # |
| --- | --- |

| Total number of accessible spaces | Click to enter # |
| --- | --- |

| Total number of van accessible spaces | Click to enter # |
| --- | --- |

| At least 1 of every 6 accessible spaces is van accessible? | ☐ Yes ☐ No |
| --- | --- |

| Accessible spaces located closest to nearest accessible route? | ☐ Yes ☐ No |
| --- | --- |

DIMENSIONS

| Width of accessible parking spaces (inches) Min. 96", All standards | Click to enter in |
| --- | --- |

| Width of access aisle (inches) Min. 60" standard ¦ Min. 96" van, ANSI/UFAS/1991 ADA ¦ Min. 60" van, 2010 ADA if space is 132" | Click to enter in |
| --- | --- |

| Width of van accessible space (inches) Min. 132" OR 96"+60" aisle, 2010 ADA ¦ Min. 96"+96" aisle, ANSI/UFAS/1991 ADA | Click to enter in |
| --- | --- |

| Width of van access aisle (inches) | Click to enter in |
| --- | --- |

| Vertical clearance for van parking/access aisles (inches) Min. 98", All standards | Click to enter in |
| --- | --- |

| Ground surface slope less than 1:48? | ☐ Yes ☐ No |
| --- | --- |

| Access aisle slope less than 1:48? | ☐ Yes ☐ No |
| --- | --- |

| Parking surface stable, firm, slip-resistant, and free of potholes or broken pavement at accessible spaces and access aisles? | ☐ Yes ☐ No |
| --- | --- |

MARKINGS & SIGNAGE

| Accessible spaces marked with lines? | ☐ Yes ☐ No |
| --- | --- |

| Access aisle marked to discourage parking in it? | ☐ Yes ☐ No |
| --- | --- |

| Vertical ISA sign present? | ☐ Yes ☐ No |
| --- | --- |

| Height of ISA signage (inches) | Click to enter in |
| --- | --- |

| ISA contrasts light-on-dark or dark-on-light? | ☐ Yes ☐ No |
| --- | --- |

| Van accessible spaces marked with lines? | ☐ Yes ☐ No |
| --- | --- |

| "Van Accessible" signs posted at van spaces? | ☐ Yes ☐ No |
| --- | --- |

| Height of Van Accessible ISA signage (inches) | Click to enter in |
| --- | --- |

| Individuals with disabilities required to wheel/walk behind parked cars? | ☐ Yes ☐ No |
| --- | --- |

| Detectable warnings at curb ramps in transit facilities? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Passenger Loading Zones

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, PASSENGER LOADING ZONES |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, PASSENGER LOADING ZONES |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Passenger loading zone provided? (If none, mark N/A and skip remaining fields) | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Zone serves which area? | e.g. main entrance drop-off, bus loop |
| --- | --- |

| Access aisle adjacent and parallel to the vehicle pull-up space? | ☐ Yes ☐ No |
| --- | --- |

| Access aisle, Width x Length Min. 60 in. wide x 20 ft long, all standards | W in.: in L ft: in |
| --- | --- |

| Vertical clearance at the zone (inches) Min. 114 in., 2010 ADA | Click to enter in |
| --- | --- |

| Access aisle marked? | ☐ Yes ☐ No |
| --- | --- |

| Aisle at same level as pull-up space (no curb between)? | ☐ Yes ☐ No |
| --- | --- |

| Where a curb separates the aisle from the accessible route, curb ramp provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Slope of pull-up space and aisle (enter denominator) Max 1:48, all standards | Click to enter 1:? |
| --- | --- |

| Zone connects to an accessible route to the entrance? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Accessible Routes / Walkways

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, ACCESSIBLE ROUTES / WALKWAYS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, ACCESSIBLE ROUTES / WALKWAYS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| At least one accessible route from arrival points to facility entrance? | ☐ Yes ☐ No |
| --- | --- |

| Inaccessible entrances marked with directional signage? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Width of public walkways (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Overhead clearance (inches) Min. 80", All standards | Click to enter in |
| --- | --- |

| 60" passing space at intervals along route? Required, 2010 ADA where route is less than 60" wide | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Path of travel stable, firm, slip-resistant, and accessible? | ☐ Yes ☐ No |
| --- | --- |

| Surface material | Click to pick |
| --- | --- |

| Walkways continuous, not interrupted by steps or abrupt level changes? | ☐ Yes ☐ No |
| --- | --- |

| Where paths cross driveways/parking lots, walkways blend to common level? | ☐ Yes ☐ No |
| --- | --- |

| Curb cuts provided at driveways and parking lots? | ☐ Yes ☐ No |
| --- | --- |

| Protruding objects present? Objects must not protrude more than 4" into path between 27"-80" above floor | ☐ Yes ☐ No |
| --- | --- |

| If yes, describe protruding objects | Description |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Curb Ramps

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, CURB RAMPS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, CURB RAMPS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Width of ramp run, not including flared sides (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Rise measurement (inches) | Click to enter in |
| --- | --- |

| Length measurement (inches) | Click to enter in |
| --- | --- |

| Running slope (enter denominator, e.g. 12 for 1:12) Max 1:12, All standards | Click to enter 1:? |
| --- | --- |

| Cross slope (enter denominator) Max 1:48 (2%), All standards | Click to enter 1:? |
| --- | --- |

| Gutter slope measurement | ratio or % |
| --- | --- |

| Curb ramp has flared sides? | ☐ Yes ☐ No |
| --- | --- |

| Flared sides ratio (enter denominator) Max 1:10, All standards | Click to enter 1:? |
| --- | --- |

| Landing provided at top of curb ramp? | ☐ Yes ☐ No |
| --- | --- |

| Landing area, Length × Width (inches) Min. 36"×36" at top, All standards | L: in W: in |
| --- | --- |

| Curb ramp placed diagonally at intersection? | ☐ Yes ☐ No |
| --- | --- |

| If yes, clear space measurement (inches) | Click to enter in |
| --- | --- |

| Raised islands in crossings? | ☐ Yes ☐ No |
| --- | --- |

| If yes, curb ramp separation, Length × Width (inches) | L: in W: in |
| --- | --- |

| Surface stable, firm, and slip-resistant? | ☐ Yes ☐ No |
| --- | --- |

| Detectable warnings provided? | ☐ Yes ☐ No |
| --- | --- |

| Detectable warning dome size and spacing | Size: in Spacing: in |
| --- | --- |

| Detectable warnings light-on-dark or dark-on-light contrast? | ☐ Yes ☐ No |
| --- | --- |

| Gratings in walking surface? | ☐ Yes ☐ No |
| --- | --- |

| Grating space exceeds 1/2" wide in one direction? | ☐ Yes ☐ No |
| --- | --- |

| Any obstructions? (List) | Description |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Stadium / Field

| SECTION DETAILS, STADIUM / FIELD |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, STADIUM / FIELD |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Accessible route to area? | ☐ Yes ☐ No |
| --- | --- |

| Route surface material | Click to pick |
| --- | --- |

| Total number of wheelchair designated seats/spaces | Click to enter # |
| --- | --- |

| Wheelchair designated seating provided? | ☐ Yes ☐ No |
| --- | --- |

| Accessible route to wheelchair designated seats? | ☐ Yes ☐ No |
| --- | --- |

| Companion seats provided with wheelchair spaces? | ☐ Yes ☐ No |
| --- | --- |

| Ramp to wheelchair seating area? (If yes, complete Ramps section) | ☐ Yes ☐ No |
| --- | --- |

| Accessible route clear width (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Entrances, Doors, and Gates

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, ENTRANCES, DOORS, AND GATES |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, ENTRANCES, DOORS, AND GATES |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

SIGNAGE

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

| Entrance marked with ISA (International Symbol of Accessibility)? | ☐ Yes ☐ No |
| --- | --- |

| ISA contrasts light-on-dark or dark-on-light? | ☐ Yes ☐ No |
| --- | --- |

| Inaccessible entrances have directional signage to accessible entrance? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

DOOR HARDWARE & DIMENSIONS

| Total number of accessible entrances | Click to enter # |
| --- | --- |

| Total number of exits | Click to enter # |
| --- | --- |

| Ramp leading to entrance? (If yes, complete Ramps section) | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Width of doorway(s) (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle, floor to handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| If not, type of hardware on door | push bar, knob, lever, etc. |
| --- | --- |

| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door opening force (pounds) Max 5 lbs interior, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height, Exterior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Threshold height, Interior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Carpeting or mats 1/2 inch high or less? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Floor level within 5 feet of door (direction door swings)? | ☐ Yes ☐ No |
| --- | --- |

| Doorway platform measurement beyond each side of door (inches) | Click to enter in |
| --- | --- |

| Sharp inclines and abrupt level changes avoided at threshold? | ☐ Yes ☐ No |
| --- | --- |

| Main entrance a fire door? | ☐ Yes ☐ No |
| --- | --- |

ALARMS

| Visual alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Light flashes clear or normal white? | ☐ Yes ☐ No |
| --- | --- |

| Audible alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Stairways and Steps

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, STAIRWAYS AND STEPS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, STAIRWAYS AND STEPS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Stair location | Click to pick |
| --- | --- |

| Steps avoid abrupt nosing? | ☐ Yes ☐ No |
| --- | --- |

| Stair width (inches) Min. 44" public stair, All standards | Click to enter in |
| --- | --- |

| Handrails provided on both sides? | ☐ Yes ☐ No |
| --- | --- |

| Height of handrails (inches) 30"-34", ANSI/UFAS ¦ 34"-38", 1991 ADA/2010 ADA | Click to enter in |
| --- | --- |

| Length of handrails (inches) | Click to enter in |
| --- | --- |

| Do steps have risers? | ☐ Yes ☐ No |
| --- | --- |

| Height of risers (inches) Max 7", All standards | Click to enter in |
| --- | --- |

| Tread depth (inches) Min. 11", All standards | Click to enter in |
| --- | --- |

| Color/tonal contrast on stair nosings? Recommended, 2010 ADA; required by many state standards | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Ramps

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, RAMPS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, RAMPS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Ramp serves which area? | e.g. Main entrance, Building B north door |
| --- | --- |

| Rise measurement, height of run (inches) | Click to enter in |
| --- | --- |

| Length measurement (inches) | Click to enter in |
| --- | --- |

| Maximum rise per run (inches) Max 30" before intermediate landing required, All standards | Click to enter in |
| --- | --- |

| Running slope (enter denominator) Max 1:12, All standards | Click to enter 1:? |
| --- | --- |

| Cross slope (enter denominator) Max 1:48 (2%), All standards | Click to enter 1:? |
| --- | --- |

| Surface stable, firm, and slip-resistant? | ☐ Yes ☐ No |
| --- | --- |

| Accessible landing at top and bottom? | ☐ Yes ☐ No |
| --- | --- |

| Top landing, Length × Width (inches) Min. 60"×60", All standards | L: in W: in |
| --- | --- |

| Bottom landing, Length × Width (inches) Min. 60"×60", All standards | L: in W: in |
| --- | --- |

| Intermediate landing between runs? | ☐ Yes ☐ No |
| --- | --- |

| Ramp changes direction? | ☐ Yes ☐ No |
| --- | --- |

| Intermediate / direction-change landing, Length × Width (inches) Min. 60"×60", All standards | L: in W: in |
| --- | --- |

| Landing subject to wet conditions? | ☐ Yes ☐ No |
| --- | --- |

| If yes, landing drainage slope (ratio) | ratio |
| --- | --- |

| Clear width of ramp between handrails (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Handrails on both sides? | ☐ Yes ☐ No |
| --- | --- |

| Height of handrails (inches) 30"-34", ANSI/UFAS ¦ 34"-38", 1991 ADA/2010 ADA | Click to enter in |
| --- | --- |

| Handrails smooth and extend beyond top and bottom of ramp? | ☐ Yes ☐ No |
| --- | --- |

| Handrails continuous? | ☐ Yes ☐ No |
| --- | --- |

| If not continuous, length of horizontal extensions (inches) | Click to enter in |
| --- | --- |

| Height of edge protection (inches) | Click to enter in |
| --- | --- |

| Ramp has switchback or dogleg? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Elevators

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, ELEVATORS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, ELEVATORS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Building has multiple stories? | ☐ Yes ☐ No |
| --- | --- |

| Elevator available and usable by individuals with physical disabilities? | ☐ Yes ☐ No |
| --- | --- |

| Unassisted access to elevator? | ☐ Yes ☐ No |
| --- | --- |

CALL CONTROLS

| Height of call button, floor to center of call button panel (inches) 15"-48", All standards | Click to enter in |
| --- | --- |

| Car call button size, diameter (inches) Min. 3/4", 2010 ADA | Click to enter in |
| --- | --- |

| Buttons easy to push or touch sensitive? | ☐ Yes ☐ No |
| --- | --- |

| Visible and audible signals at each hoist way entrance? | ☐ Yes ☐ No |
| --- | --- |

| Hall lantern provided? | ☐ Yes ☐ No |
| --- | --- |

| Call buttons have visible/audible signals when call registered? | ☐ Yes ☐ No |
| --- | --- |

| Audible signals: once for up, twice for down (or verbal annunciator)? | ☐ Yes ☐ No |
| --- | --- |

HOIST WAY & CAR

| Floor designations, raised characters and Braille on both door jambs? | ☐ Yes ☐ No |
| --- | --- |

| Floor designation character height (inches) Min. 2", All standards | Click to enter in |
| --- | --- |

| Type of elevator entrance door | Click to pick |
| --- | --- |

| Width of elevator entrance door (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Door automatically stops/reopens if obstructed, remains open min. 20 seconds? | ☐ Yes ☐ No |
| --- | --- |

| Door reopening device sensitive to light touch or contact? | ☐ Yes ☐ No |
| --- | --- |

| Car interior, Width × Depth (inches) Min. 51"W × 51"D center door ¦ 68"W × 51"D side door, 2010 ADA | W: in D: in |
| --- | --- |

| Width of car door (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Emergency alarm/stop at bottom of panel? | ☐ Yes ☐ No |
| --- | --- |

| Height of emergency communications, floor to bottom of panel (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| If not accessible, accessible elevator clearly identified with ISA? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Lifts

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, LIFTS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, LIFTS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Platform lift provides unassisted entry and exit? | ☐ Yes ☐ No |
| --- | --- |

| Walkways have leveled platform to runway clearance? | ☐ Yes ☐ No |
| --- | --- |

| Clear space for approach, Length × Width (inches) | L: in W: in |
| --- | --- |

| Height of controls, floor to middle of control panel (inches) 15"-48", All standards | Click to enter in |
| --- | --- |

| Low-energy, power-operated doors or gates? | ☐ Yes ☐ No |
| --- | --- |

| Doors remain open minimum 20 seconds? | ☐ Yes ☐ No |
| --- | --- |

| Width of end doors and gates (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Width of side doors and gates (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Lift interior, Width × Depth (inches) Min. 32"W × 48"D, All standards | W: in D: in |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Accessible Means of Egress / Areas of Rescue Assistance

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, ACCESSIBLE MEANS OF EGRESS / AREAS OF RESCUE ASSISTANCE |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, ACCESSIBLE MEANS OF EGRESS / AREAS OF RESCUE ASSISTANCE |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Accessible means of egress provided from all occupied areas? | ☐ Yes ☐ No |
| --- | --- |

| Number of accessible means of egress Same number as required exits, up to code limits | Click to enter # |
| --- | --- |

| Areas of Rescue Assistance / areas of refuge designated (multi-story buildings)? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Number of wheelchair spaces in each area of rescue assistance Min. 30 in. x 48 in. per space | Click to enter # |
| --- | --- |

| Two-way communication system at each area of rescue assistance? | ☐ Yes ☐ No |
| --- | --- |

| Communication system tested and operational? | ☐ Yes ☐ No |
| --- | --- |

| Signage identifying areas of rescue assistance (with ISA)? | ☐ Yes ☐ No |
| --- | --- |

| Illuminated exit signage on accessible egress routes? | ☐ Yes ☐ No |
| --- | --- |

| Egress routes free of steps and barriers, or an evacuation alternative provided? | ☐ Yes ☐ No |
| --- | --- |

| Evacuation procedures address mobility impairments? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Rooms & Offices

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, ROOMS & OFFICES |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, ROOMS & OFFICES |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Room Name / Type (e.g. Wellness Center, Career Center, Counseling Office) | Enter room name or type |
| --- | --- |

| Room Number(s) | e.g. 101, B-12 |
| --- | --- |

| Room/office on accessible route? | ☐ Yes ☐ No |
| --- | --- |

SIGNAGE

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

DOORS

| Number of entrances and exits | Click to enter # |
| --- | --- |

| Fire door or exterior hinged door? | ☐ Yes ☐ No |
| --- | --- |

| Width of doorway(s) (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle, floor to handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| If not, type of hardware on door | push bar, knob, lever, etc. |
| --- | --- |

| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door opening force (pounds) Max 5 lbs interior, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height, Exterior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Threshold height, Interior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Carpeting or mats 1/2 inch high or less? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

COUNTERS & WORKSTATIONS

| Accessible Sales/Services reception countertop? Requires additional measurements | ☐ Yes ☐ No |
| --- | --- |

| Forward reach approach (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Parallel approach (inches) | Click to enter in |
| --- | --- |

| Accessible reception countertop? | ☐ Yes ☐ No |
| --- | --- |

| Height of reception countertop (inches) Max 34", ANSI/UFAS ¦ Max 36", 1991 ADA/2010 ADA | Click to enter in |
| --- | --- |

| Countertop length × width (inches) | L: in W: in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D, All standards | H: in W: in D: in |
| --- | --- |

| Supplies accessible by persons with disabilities? | ☐ Yes ☐ No |
| --- | --- |

ALARMS

| Visual alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Light flashes clear or normal white? | ☐ Yes ☐ No |
| --- | --- |

| Audible alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Medical and First-Aid Areas (including Nurse's Office)

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, MEDICAL AND FIRST-AID AREAS (INCLUDING NURSE'S OFFICE) |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, MEDICAL AND FIRST-AID AREAS (INCLUDING NURSE'S OFFICE) |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Room name / number | e.g. Nurse's Office, Health Office, First-Aid Room |
| --- | --- |

| Medical / first-aid area on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Width of doorway (inches) Min. 32 in., all standards | Click to enter in |
| --- | --- |

| Height of door handle (inches) Max 48 in., all standards | Click to enter in |
| --- | --- |

| Door opening force (pounds) Max 5 lbs, 1991 ADA / UFAS / 2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Turning space, diameter (inches) Min. 60 in., all standards | Click to enter in |
| --- | --- |

| Height of examination cot / bed surface (inches) 17 in. to 19 in. supports transfer | Click to enter in |
| --- | --- |

| 30 in. x 48 in. clear floor space alongside the cot / bed? | ☐ Yes ☐ No |
| --- | --- |

| Sink accessible? Rim max 34 in.; knee clearance min. 27 in. H / 30 in. W / 19 in. D | ☐ Yes ☐ No |
| --- | --- |

| Height of supplies and operable parts (inches) Max 48 in. forward reach, all standards | Click to enter in |
| --- | --- |

| Privacy accommodations (curtained or partitioned area) accessible? | ☐ Yes ☐ No |
| --- | --- |

| Identification signage with raised characters and Braille? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Cafeteria

| SECTION DETAILS, CAFETERIA |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, CAFETERIA |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

SIGNAGE

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

ENTRANCE & DOORS

| Number of entrances and exits | Click to enter # |
| --- | --- |

| Cafeteria on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Ramps leading to cafeteria? (If yes, complete Ramps section) | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Width of doorway(s) (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle, floor to handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| If not, type of hardware on door | push bar, knob, lever, etc. |
| --- | --- |

| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door opening force (pounds) Max 5 lbs interior, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height, Exterior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Threshold height, Interior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Carpeting or mats 1/2 inch high or less? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

ALARMS

| Visual alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Light flashes clear or normal white? | ☐ Yes ☐ No |
| --- | --- |

| Audible alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| --- | --- |

INDOOR FIXED SEATING

| Total number of fixed accessible seats indoors | Click to enter # |
| --- | --- |

| Seats and tables on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Wheelchair seating space at fixed tables/counters? | ☐ Yes ☐ No |
| --- | --- |

| Height of fixed tables/countertops (inches) 28"-34", All standards | Click to enter in |
| --- | --- |

| Wheelchair space, Width × Depth (inches) Min. 30"×48", All standards | W: in D: in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D | H: in W: in D: in |
| --- | --- |

| Accessible route between food line, seating, and exit? | ☐ Yes ☐ No |
| --- | --- |

OUTDOOR FIXED SEATING

| Total number of fixed accessible seats outdoors | Click to enter # |
| --- | --- |

| Seats and tables on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Height of outdoor fixed tables/countertops (inches) 28"-34", All standards | Click to enter in |
| --- | --- |

| Outdoor surface material | Click to pick |
| --- | --- |

FOOD SERVICE LINE(S)

Measure all food service lines including main line, à la carte, and any student store lines.

| Total number of food lines | Click to enter # |
| --- | --- |

| Food service line aisle width (inches) Min. 36" clear, All standards | Click to enter in |
| --- | --- |

| Height of food line counter / tray slide (inches) Max 34", ANSI/UFAS ¦ Max 36", 1991 ADA/2010 ADA | Click to enter in |
| --- | --- |

| Height of sneeze guard / food display, lowest accessible item (inches) Max 48" forward reach, All standards | Click to enter in |
| --- | --- |

| Self-service items (utensils, condiments, trays) within reach range? Max 48" forward / 54" side, ANSI/UFAS/1991 ADA ¦ Max 48" side, 2010 ADA | ☐ Yes ☐ No |
| --- | --- |

| Height of highest self-service item, operable part (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Staff-assisted alternative available if self-service not accessible? | ☐ Yes ☐ No |
| --- | --- |

CHECKOUT COUNTER(S)

| Total number of checkout aisles | Click to enter # |
| --- | --- |

| Number of accessible checkout aisles Min. 1 required, All standards | Click to enter # |
| --- | --- |

| Checkout aisle width (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Height of accessible checkout counter (inches) Max 34", ANSI/UFAS ¦ Max 36", 1991 ADA/2010 ADA | Click to enter in |
| --- | --- |

| Width of lowered counter section (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D | H: in W: in D: in |
| --- | --- |

| Approach type at checkout | Click to pick |
| --- | --- |

| Height of tableware and condiment areas (inches) Max 48" forward reach, All standards | Click to enter in |
| --- | --- |

TRAY RETURN STATION

| Tray return station provided? | ☐ Yes ☐ No |
| --- | --- |

| Tray return on accessible route? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Height of tray return slot (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Clear floor space in front, Width × Depth (inches) Min. 30"×48", All standards | W: in D: in |
| --- | --- |

SELF-SERVICE BEVERAGE STATION

| Self-service beverage station provided? | ☐ Yes ☐ No |
| --- | --- |

| Station on accessible route? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Height of highest operable part (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Clear floor space, Width × Depth (inches) Min. 30"×48", All standards | W: in D: in |
| --- | --- |

| Operable parts require tight grasping/pinching/twisting? | ☐ Yes ☐ No |
| --- | --- |

| Force to activate exceeds 5 lbs? | ☐ Yes ☐ No |
| --- | --- |

VENDING MACHINES

| Vending machines provided? | ☐ Yes ☐ No |
| --- | --- |

| Vending machines on accessible route? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Height of highest operable part (inches) Max 48" forward reach, All standards | Click to enter in |
| --- | --- |

| Clear floor space in front, Width × Depth (inches) Min. 30"×48", All standards | W: in D: in |
| --- | --- |

| Force to activate exceeds 5 lbs? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Library

| SECTION DETAILS, LIBRARY |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, LIBRARY |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

SIGNAGE

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

ENTRANCE & DOORS

| Number of entrances and exits | Click to enter # |
| --- | --- |

| Library on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Ramps leading to library? (If yes, complete Ramps section) | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Overhead clearance of accessible route (inches) Min. 80", All standards | Click to enter in |
| --- | --- |

| Width of doorway(s) (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle, floor to handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| If not, type of hardware on door | push bar, knob, lever, etc. |
| --- | --- |

| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door opening force (pounds) Max 5 lbs interior, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height, Exterior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Threshold height, Interior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Carpeting or mats 1/2 inch high or less? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

ALARMS

| Visual alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Light flashes clear or normal white? | ☐ Yes ☐ No |
| --- | --- |

| Audible alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| --- | --- |

SEATING & WORKSTATIONS

| Seating space provided for wheelchair users at fixed tables/counters? | ☐ Yes ☐ No |
| --- | --- |

| Seats and tables on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Number of accessible seats provided | Click to enter # |
| --- | --- |

| Height of fixed tables/countertops (inches) 28"-34", All standards | Click to enter in |
| --- | --- |

| Aisle space width (inches) Min. 36", All standards ¦ Min. 60" passing space at intervals, 2010 ADA | Click to enter in |
| --- | --- |

| Turning space in reading areas, diameter (inches) Min. 60", All standards | Click to enter in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D, All standards | H: in W: in D: in |
| --- | --- |

| Accessible desk/workstations have sufficient clear space? | ☐ Yes ☐ No |
| --- | --- |

| If insufficient, alternative option available for students? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

COMPUTER WORKSTATIONS & ADAPTIVE TECHNOLOGY

| Computer workstations available? | ☐ Yes ☐ No |
| --- | --- |

| Accessible computer workstation(s) provided? Required if workstations provided, All standards | ☐ Yes ☐ No |
| --- | --- |

| Height of accessible workstation surface (inches) 28"-34", All standards | Click to enter in |
| --- | --- |

| Adaptive technology available? (screen reader, magnification, etc.) | ☐ Yes ☐ No |
| --- | --- |

| If yes, describe adaptive technology available | e.g. screen reader, magnification software, large-print keyboard |
| --- | --- |

CHECKOUT COUNTER

| Number of accessible checkout areas | Click to enter # |
| --- | --- |

| Height of checkout counter top, floor to surface (inches) Max 34", ANSI/UFAS ¦ Max 36", 1991 ADA/2010 ADA | Click to enter in |
| --- | --- |

| Width of lowered counter section (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Counter top depth (inches) | Click to enter in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D | H: in W: in D: in |
| --- | --- |

| Forward reach approach (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Parallel approach available? | ☐ Yes ☐ No |
| --- | --- |

BOOK DROP

Measuring the Book Drop: measure from finished floor to the bottom edge of the drop slot opening, in inches. Measure clear floor space in front (W×D). Test force required to activate in lbs. Note approach type and interior/exterior.

| Book drop provided? | ☐ Yes ☐ No |
| --- | --- |

| Book drop on accessible route? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Height of drop slot, floor to bottom edge of slot opening (inches) Max 48" forward reach, All standards ¦ Max 54" side, ANSI/UFAS/1991 ADA ¦ Max 48" side, 2010 ADA | Click to enter in |
| --- | --- |

| Clear floor space in front, Width × Depth (inches) Min. 30"×48", All standards | W: in D: in |
| --- | --- |

| Force required to activate/open book drop (pounds) Max 5 lbs, All standards | Click to enter lbs |
| --- | --- |

| Operable parts require tight grasping, pinching, or twisting? | ☐ Yes ☐ No |
| --- | --- |

| Approach type | Click to pick |
| --- | --- |

| Book drop location | Click to pick |
| --- | --- |

CARD CATALOG & STACK AISLES

| Card catalog aisle width (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Card catalog operable part height (inches) Max 48" forward reach, All standards | Click to enter in |
| --- | --- |

| Stack aisle width (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Overhead clearance in stack aisles (inches) Min. 80", All standards | Click to enter in |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

CTE Classroom(s)

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, CTE CLASSROOM(S) |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, CTE CLASSROOM(S) |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Room Number(s) | e.g. 204, Shop B |
| --- | --- |

| Program / Course Name | e.g. Auto Tech, Culinary Arts |
| --- | --- |

| Classroom on accessible route? | ☐ Yes ☐ No |
| --- | --- |

SIGNAGE

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

DOORS

| Number of entrances and exits | Click to enter # |
| --- | --- |

| Width of doorway(s) (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle, floor to handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| If not, type of hardware on door | push bar, knob, lever, etc. |
| --- | --- |

| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door opening force (pounds) Max 5 lbs interior, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height, Exterior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Threshold height, Interior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Carpeting or mats 1/2 inch high or less? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

SEATING & WORKSTATIONS

| Wheelchair seating at fixed tables/counters? | ☐ Yes ☐ No |
| --- | --- |

| Number of accessible seats | Click to enter # |
| --- | --- |

| Height of fixed tables/countertops (inches) 28"-34", All standards | Click to enter in |
| --- | --- |

| Aisle space width (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D | H: in W: in D: in |
| --- | --- |

| Accessible desk/workstations have sufficient clearance? | ☐ Yes ☐ No |
| --- | --- |

| If insufficient, alternative option available? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Supplies accessible by persons with disabilities? | ☐ Yes ☐ No |
| --- | --- |

ALARMS

| Visual alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Light flashes clear or normal white? | ☐ Yes ☐ No |
| --- | --- |

| Audible alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

CTE Labs and Shops

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, CTE LABS AND SHOPS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, CTE LABS AND SHOPS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Room Number(s) | e.g. Shop 1, Lab B |
| --- | --- |

| Program / Course Name | e.g. Welding, Biology Lab |
| --- | --- |

| Lab/Shop on accessible route? | ☐ Yes ☐ No |
| --- | --- |

SIGNAGE

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

DOORS

| Number of entrances and exits | Click to enter # |
| --- | --- |

| Width of doorway(s) (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle, floor to handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| If not, type of hardware on door | push bar, knob, lever, etc. |
| --- | --- |

| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door opening force (pounds) Max 5 lbs interior, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height, Exterior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Threshold height, Interior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Carpeting or mats 1/2 inch high or less? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

WORKSTATIONS

| Wheelchair seating at fixed tables/counters? | ☐ Yes ☐ No |
| --- | --- |

| Height of fixed tables/countertops (inches) 28"-34", All standards | Click to enter in |
| --- | --- |

| Aisle space width (inches) Min. 36", All standards | Click to enter in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D | H: in W: in D: in |
| --- | --- |

| Accessible desk/workstations have sufficient clearance? | ☐ Yes ☐ No |
| --- | --- |

| Supplies accessible by persons with disabilities? | ☐ Yes ☐ No |
| --- | --- |

| Height of soap dispenser (floor to operable part, inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Height of paper towel dispenser (floor to operable part, inches) Max 48", All standards | Click to enter in |
| --- | --- |

ALARMS

| Visual alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Light flashes clear or normal white? | ☐ Yes ☐ No |
| --- | --- |

| Audible alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Portable / Temporary Classrooms

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, PORTABLE / TEMPORARY CLASSROOMS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, PORTABLE / TEMPORARY CLASSROOMS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Portable / building number | e.g. P-3 |
| --- | --- |

| Year installed | Click to enter yyyy |
| --- | --- |

| Portable on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Ramp provided to entrance? (If yes, also complete the Ramps section) | ☐ Yes ☐ No |
| --- | --- |

| Ramp running slope (enter denominator) Max 1:12, all standards | Click to enter 1:? |
| --- | --- |

| Landing at door, Length x Width (inches) Min. 60 in. x 60 in., all standards | L: in W: in |
| --- | --- |

| Width of doorway (inches) Min. 32 in., all standards | Click to enter in |
| --- | --- |

| Height of door handle (inches) Max 48 in., all standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| Door opening force (pounds) Max 5 lbs, 1991 ADA / UFAS / 2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height (inches) Max 1/2 in., all standards | Click to enter in |
| --- | --- |

| Handrails on both sides of ramp where rise exceeds 6 in.? | ☐ Yes ☐ No |
| --- | --- |

| Height of handrails (inches) 30 in. to 34 in., ANSI / UFAS ¦ 34 in. to 38 in., 1991 ADA / 2010 ADA | Click to enter in |
| --- | --- |

| Fire / emergency egress from portable accessible? | ☐ Yes ☐ No |
| --- | --- |

| Visual and audible alarms provided? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Gymnasium / Auditorium / Weight Room

| SECTION DETAILS, GYMNASIUM / AUDITORIUM / WEIGHT ROOM |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, GYMNASIUM / AUDITORIUM / WEIGHT ROOM |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

Complete for Gymnasium, Auditorium, Weight Room, or other large assembly space. If the campus has separate Gym and Auditorium buildings, note both in Additional Information or use separate printed copies.

| Space Type | Click to pick |
| --- | --- |

| Room Number(s) | e.g. Gym A, Aud 1 |
| --- | --- |

SIGNAGE

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

ENTRANCE & DOORS

| Space on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Number of entrances and exits | Click to enter # |
| --- | --- |

| Width of doorway(s) (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle, floor to handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No |
| --- | --- |

| If not, type of hardware on door | push bar, knob, lever, etc. |
| --- | --- |

| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Door opening force (pounds) Max 5 lbs interior, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Threshold height, Exterior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Threshold height, Interior (inches) Max 1/2", All standards | Click to enter in |
| --- | --- |

| Carpeting or mats 1/2 inch high or less? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

ALARMS

| Visual alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Light flashes clear or normal white? | ☐ Yes ☐ No |
| --- | --- |

| Audible alarm provided? | ☐ Yes ☐ No |
| --- | --- |

| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| --- | --- |

WHEELCHAIR SEATING

| Total number of all seating | Click to enter # |
| --- | --- |

| Total number of wheelchair spaces Required ratio varies by capacity, 2010 ADA §221 | Click to enter # |
| --- | --- |

| Wheelchair seating in different locations throughout venue? | ☐ Yes ☐ No |
| --- | --- |

| Companion seat provided with each wheelchair space? | ☐ Yes ☐ No |
| --- | --- |

| Accessible route to wheelchair designated seats? | ☐ Yes ☐ No |
| --- | --- |

| Ramp to seating area? (If yes, complete Ramps section) | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Line of sight for wheelchair spaces comparable to ambulatory seating? | ☐ Yes ☐ No |
| --- | --- |

| Wheelchair space dimensions, Width × Depth (inches) Min. 36"W × 48"D, All standards | W: in D: in |
| --- | --- |

| Can supplies be accessed by persons with disabilities? | ☐ Yes ☐ No |
| --- | --- |

ASSISTIVE LISTENING SYSTEMS

| Audio amplification system provided? | ☐ Yes ☐ No |
| --- | --- |

| Adequate number of assistive listening systems (ALS) provided? Required where audible communications are integral to use of space, All standards | ☐ Yes ☐ No |
| --- | --- |

| Type of assistive listening system | Click to pick |
| --- | --- |

| Number of receivers available | Click to enter # |
| --- | --- |

| If fixed audio amplification, distance from stage (feet) | Click to enter ft |
| --- | --- |

| Signage indicating availability of assistive listening devices? | ☐ Yes ☐ No |
| --- | --- |

FIXED SEATING / TABLES (WEIGHT ROOM / OTHER)

| Wheelchair seating space at fixed tables/counters? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Number of accessible seats provided | Click to enter # |
| --- | --- |

| Height of fixed tables/countertops (inches) 28"-34", All standards | Click to enter in |
| --- | --- |

| Wheelchair space, Width × Depth (inches) Min. 30"×48" | W: in D: in |
| --- | --- |

| Knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D | H: in W: in D: in |
| --- | --- |

| Accessible desk/workstations have sufficient clearance? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Swimming Pools / Aquatic Areas

| SECTION DETAILS, SWIMMING POOLS / AQUATIC AREAS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, SWIMMING POOLS / AQUATIC AREAS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Swimming pool or aquatic facility present? (If none, mark N/A and skip remaining fields) | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Pool type / use | e.g. competition pool, instruction pool, spa |
| --- | --- |

| Pool wall length (linear feet) Determines required number of accessible means of entry, 2010 ADA 242 | Click to enter ft |
| --- | --- |

| Accessible means of entry provided? Pools with less than 300 linear ft of wall: min. 1 (pool lift or sloped entry); 300 linear ft or more: min. 2, at least one a pool lift or sloped entry, 2010 ADA 242 | ☐ Yes ☐ No |
| --- | --- |

| Primary accessible entry type | Click to pick |
| --- | --- |

| Pool lift available and operable during all open hours? | ☐ Yes ☐ No |
| --- | --- |

| Lift seat height above deck (inches) 16 in. to 19 in., 2010 ADA | Click to enter in |
| --- | --- |

| Accessible route to the pool deck? | ☐ Yes ☐ No |
| --- | --- |

| Deck surface stable, firm, and slip-resistant? | ☐ Yes ☐ No |
| --- | --- |

| Accessible route connects deck, entries, and locker/shower facilities? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Locker Rooms

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, LOCKER ROOMS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, LOCKER ROOMS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Locker Room Type | Click to pick |
| --- | --- |

| Building No. |  |
| --- | --- |

| Locker room on accessible route? | ☐ Yes ☐ No |
| --- | --- |

SIGNAGE & ENTRY DOOR

| Identification signage provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of signage, floor to centerline (inches) 60" centerline, All standards | Click to enter in |
| --- | --- |

| Height of characters on signage (inches) | Click to enter in |
| --- | --- |

| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| --- | --- |

| Non-glare finish on signs? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

| Width of entrance doorway (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door opening force (pounds) Max 5 lbs, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

INTERIOR MANEUVERING

| Locker room has foyer? | ☐ Yes ☐ No |
| --- | --- |

| Measurement between doors in foyer (inches) | Click to enter in |
| --- | --- |

| Turning space, diameter (inches) Min. 60", All standards | Click to enter in |
| --- | --- |

| Doors swing into turning spaces? | ☐ Yes ☐ No |
| --- | --- |

LOCKERS & AMENITIES

| Number of accessible lockers At least 5% of each type, 2010 ADA §225 | Click to enter # |
| --- | --- |

| Height of accessible locker operable parts (inches) Max 48" forward reach, All standards | Click to enter in |
| --- | --- |

| Mirror provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of mirror, floor to reflecting edge (inches) Max 40", All standards | Click to enter in |
| --- | --- |

| Accessible benches provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of benches, floor to top (inches) 17"-19", All standards | Click to enter in |
| --- | --- |

| Depth of benches (inches) Min. 20"-24", All standards | Click to enter in |
| --- | --- |

| Back support on benches? | ☐ Yes ☐ No |
| --- | --- |

| Clear floor space adjacent to bench (inches) Min. 30"×48", All standards | Click to enter in |
| --- | --- |

SHOWER ROOMS

| Showers in use? | ☐ Yes ☐ No |
| --- | --- |

| Number of showers | Click to enter # |
| --- | --- |

| Accessible shower provided? | ☐ Yes ☐ No |
| --- | --- |

| Shower type | Click to pick |
| --- | --- |

| Accessible shower, Width × Depth (inches) Transfer: 36"×36" ¦ Roll-in: 60"×30" min, 2010 ADA | W: in D: in |
| --- | --- |

| Shower has curb? | ☐ Yes ☐ No |
| --- | --- |

| Height of curb in shower (inches) Max 1/2", All standards (transfer shower) | Click to enter in |
| --- | --- |

| Shower grab bars provided? | ☐ Yes ☐ No |
| --- | --- |

| Location of shower grab bars | e.g. side wall, back wall |
| --- | --- |

| Height of shower grab bars, floor to centerline (inches) 33"-36", All standards | Click to enter in |
| --- | --- |

| Length of shower grab bars (inches) | Click to enter in |
| --- | --- |

| Accessible shower seat provided? | ☐ Yes ☐ No |
| --- | --- |

| Height of shower seat, floor to top (inches) 17"-19", All standards | Click to enter in |
| --- | --- |

| Handheld shower/spray unit provided? | ☐ Yes ☐ No |
| --- | --- |

| Handheld shower hose length (inches) Min. 59", 2010 ADA | Click to enter in |
| --- | --- |

RESTROOM AMENITIES

| Separate restroom entrance? | ☐ Yes ☐ No |
| --- | --- |

| Width of restroom entrance (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Turning space, diameter (inches) Min. 60", All standards | Click to enter in |
| --- | --- |

| Hot water available? | ☐ Yes ☐ No |
| --- | --- |

| Hot water pipes under sink covered? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Height of sink, floor to rim (inches) Max 34", All standards | Click to enter in |
| --- | --- |

| Lavatory knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D | H: in W: in D: in |
| --- | --- |

| Height of soap dispenser (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Height of mirror, floor to reflecting edge (inches) Max 40", All standards | Click to enter in |
| --- | --- |

| Height of paper towel dispenser (inches) Max 48", All standards | Click to enter in |
| --- | --- |

ACCESSIBLE STALL

| Designated accessible stall? | ☐ Yes ☐ No |
| --- | --- |

| Width of accessible stall (inches) Min. 60", All standards | Click to enter in |
| --- | --- |

| Depth of accessible stall (inches) Min. 56", All standards | Click to enter in |
| --- | --- |

| Stall door swings out? | ☐ Yes ☐ No |
| --- | --- |

| Width of stall door (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Grab bars in accessible stall? | ☐ Yes ☐ No |
| --- | --- |

| Location of grab bars | Click to pick |
| --- | --- |

| Height of grab bars, floor to centerline (inches) 33"-36", All standards | Click to enter in |
| --- | --- |

| Height of toilet seat, floor to seat edge (inches) 17"-19", All standards | Click to enter in |
| --- | --- |

| Height of flush controls, floor to controls (inches) Max 44", All standards | Click to enter in |
| --- | --- |

| Flush valve on transfer/open side of toilet? | ☐ Yes ☐ No |
| --- | --- |

| Height of toilet paper dispenser (inches) | Click to enter in |
| --- | --- |

| Urinal height, floor to rim (inches) Max 17", All standards | Click to enter in |
| --- | --- |

| Width of space around urinal (inches) Min. 30", All standards | Click to enter in |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Restrooms (Male / Female / Non-Gender Specific)

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, RESTROOMS (MALE / FEMALE / NON-GENDER SPECIFIC) |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, RESTROOMS (MALE / FEMALE / NON-GENDER SPECIFIC) |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Restroom Type | Click to pick |
| --- | --- |

| Building No. |  |
| --- | --- |

| Restroom on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| Total number of accessible restrooms in building | Click to enter # |
| --- | --- |

| If no accessible restroom in building, directional signage to nearest? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

ENTRY DOOR & SIGNAGE

| Accessible door signage (ISA)? | ☐ Yes ☐ No |
| --- | --- |

| Height of accessible signage, floor to centerline (inches) 60", All standards (57"-63" range) | Click to enter in |
| --- | --- |

| ISA contrasts light-on-dark or dark-on-light? | ☐ Yes ☐ No |
| --- | --- |

| Characters raised and accompanied by Grade II Braille? | ☐ Yes ☐ No |
| --- | --- |

| Signage mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| --- | --- |

| Entrance doors swing out? | ☐ Yes ☐ No |
| --- | --- |

| Width of entrance doorway (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Height of door handle (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Door opening force (pounds) Max 5 lbs, 1991 ADA/UFAS/2010 ADA ¦ Max 8.5 lbs, ANSI | Click to enter lbs |
| --- | --- |

| Restroom has foyer? | ☐ Yes ☐ No |
| --- | --- |

| Measurement between doors in foyer (inches) | Click to enter in |
| --- | --- |

INTERIOR MANEUVERING

| Turning space, diameter (inches) Min. 60" diameter OR T-turn, All standards | Click to enter in |
| --- | --- |

AMENITIES

| Hot water available? | ☐ Yes ☐ No |
| --- | --- |

| Hot water pipes under sink covered? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Height of sink, floor to rim (inches) Max 34", All standards | Click to enter in |
| --- | --- |

| Sink usable by wheelchair users? | ☐ Yes ☐ No |
| --- | --- |

| Lavatory knee clearance, H / W / D (inches) Min. 27"H / 30"W / 19"D, All standards | H: in W: in D: in |
| --- | --- |

| Height of soap dispenser (floor to operable part, inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Height of mirror, floor to reflecting edge (inches) Max 40", All standards | Click to enter in |
| --- | --- |

| Height of paper towel dispenser (floor to operable part, inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Height of automatic hand dryer (floor to operable part, inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Racks, dispensers, disposal units not protruding into walkways? | ☐ Yes ☐ No |
| --- | --- |

URINALS

| Height of urinal, floor to rim (inches) Max 17", All standards | Click to enter in |
| --- | --- |

| Width of space around urinal (inches) Min. 30", All standards | Click to enter in |
| --- | --- |

| Depth of space around urinal (inches) Min. 48", All standards | Click to enter in |
| --- | --- |

ACCESSIBLE STALL

| Designated accessible stall? | ☐ Yes ☐ No |
| --- | --- |

| Width of accessible stall (inches) Min. 60", All standards | Click to enter in |
| --- | --- |

| Depth of accessible stall (inches) Min. 56", All standards | Click to enter in |
| --- | --- |

| Stall door swings out? | ☐ Yes ☐ No |
| --- | --- |

| Width of stall door (inches) Min. 32", All standards | Click to enter in |
| --- | --- |

| Grab bars in accessible stall? | ☐ Yes ☐ No |
| --- | --- |

| Location of grab bars | Click to pick |
| --- | --- |

| Height of grab bars, floor to centerline (inches) 33"-36", All standards | Click to enter in |
| --- | --- |

| Length of grab bars (inches) Side wall: min. 40" ¦ Rear wall: min. 36" | Click to enter in |
| --- | --- |

| Depth of grab bars from wall (inches) | Click to enter in |
| --- | --- |

| Height of accessible toilet seat, floor to seat edge (inches) 17"-19", All standards | Click to enter in |
| --- | --- |

| Height of flush controls, floor to controls (inches) Max 44", All standards | Click to enter in |
| --- | --- |

| Flush valve on transfer/open side of toilet? | ☐ Yes ☐ No |
| --- | --- |

| Obstructions to reach toilet paper dispenser? | ☐ Yes ☐ No |
| --- | --- |

| Toilet paper dispenser operable with one hand? | ☐ Yes ☐ No |
| --- | --- |

| Dispenser allows continuous flow of paper? | ☐ Yes ☐ No |
| --- | --- |

| Height of toilet paper dispenser (inches) | Click to enter in |
| --- | --- |

| Location of toilet tissue dispenser | e.g. side wall below grab bar |
| --- | --- |

| Coat hooks/shelves provided? | ☐ Yes ☐ No |
| --- | --- |

| Coat hook height, floor to hook (inches) Max 48" forward reach, All standards | Click to enter in |
| --- | --- |

| Type of reach for coat hook | Click to pick |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Accessible Drinking Fountains

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, ACCESSIBLE DRINKING FOUNTAINS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, ACCESSIBLE DRINKING FOUNTAINS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Drinking fountains provided? | ☐ Yes ☐ No |
| --- | --- |

| Number of fountains | Click to enter # |
| --- | --- |

| Number of accessible fountains | Click to enter # |
| --- | --- |

| At least 50% of fountains accessible (spout max 36" from floor)? | ☐ Yes ☐ No |
| --- | --- |

| Fountain on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| 30"×48" clear floor space in front? | ☐ Yes ☐ No |
| --- | --- |

| Height of spout, floor to spout outlet (inches) Max 36", All standards | Click to enter in |
| --- | --- |

| Height of water flow, spout to top of arc (inches) Min. 4", All standards | Click to enter in |
| --- | --- |

| Water flow within 4" of spout towards front of fountain? | ☐ Yes ☐ No |
| --- | --- |

| Up-front spout and control? | ☐ Yes ☐ No |
| --- | --- |

| Controls mounted on front or side? | Click to pick |
| --- | --- |

| Controls operable with closed fist? | ☐ Yes ☐ No |
| --- | --- |

| Operable parts require tight grasping/pinching/twisting? | ☐ Yes ☐ No |
| --- | --- |

| Force to activate exceeds 5 lbs? | ☐ Yes ☐ No |
| --- | --- |

| Fountain protrudes into corridors or traffic ways? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Water Coolers & Bottle Fillers

If this area exists in more than one location, click anywhere inside the block below and use the plus (+) control at its edge to add another blank copy for each additional location.

| SECTION DETAILS, WATER COOLERS & BOTTLE FILLERS |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, WATER COOLERS & BOTTLE FILLERS |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Bottle filler station available? | ☐ Yes ☐ No |
| --- | --- |

| Station on accessible route? | ☐ Yes ☐ No |
| --- | --- |

| 30"×48" clear floor space in front? | ☐ Yes ☐ No |
| --- | --- |

| Station hand operated? | ☐ Yes ☐ No |
| --- | --- |

| Station sensor operated? | ☐ Yes ☐ No |
| --- | --- |

| Controls mounted on front or side? | Click to pick |
| --- | --- |

| Height of highest operable part (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Controls operable with closed fist? | ☐ Yes ☐ No |
| --- | --- |

| Operable parts require tight grasping/pinching/twisting? | ☐ Yes ☐ No |
| --- | --- |

| Force to activate exceeds 5 lbs? | ☐ Yes ☐ No |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

Public Telephones

| SECTION DETAILS, PUBLIC TELEPHONES |  |
| --- | --- |
| Location / Building Name: | Click to type the location |
| Year Built: | Click to enter the year built |
| Have any alterations or ADA modifications been made since original construction? If Yes, complete the Alterations Log below. | - Pick one - |

| ALTERATIONS LOG, PUBLIC TELEPHONES |  |
| --- | --- |
| List each element altered and the date of the alteration. Add more rows by tabbing in the last cell. |  |
| ELEMENT ALTERED | DATE ALTERED |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |
| (e.g., "Toilet seat") | Click to pick date |

| Public telephone provided? (If No, mark N/A and skip remaining fields) | ☐ Yes ☐ No |
| --- | --- |

| Clear floor space, Width × Length (inches) Min. 30"×48", All standards | W: in L: in |
| --- | --- |

| Protruding objects? | ☐ Yes ☐ No |
| --- | --- |

| Push button controls? | ☐ Yes ☐ No |
| --- | --- |

| Cord length, telephone to handset (inches) Min. 29", All standards | Click to enter in |
| --- | --- |

| Approach type | Click to pick |
| --- | --- |

| Distance from front edge of counter to face of telephone unit (inches) | Click to enter in |
| --- | --- |

| Highest operable part within 15"-48"? (54" max if side approach) | ☐ Yes ☐ No |
| --- | --- |

| Telephone hearing aid compatible? | ☐ Yes ☐ No |
| --- | --- |

| Volume control provided? | ☐ Yes ☐ No |
| --- | --- |

| If more than 4 pay telephones, TTY available? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Height of TTY keypad touch surface (inches) Max 48", All standards | Click to enter in |
| --- | --- |

| Telephones with TTY equipped with shelf and electrical outlet? | ☐ Yes ☐ No ☐ N/A |
| --- | --- |

| Additional Information / Observations | Narrative observations, photo references, corrective actions noted... |
| --- | --- |

# Glossary of Terms

As defined by 2010 ADA Standards

Note: The terms defined below are only applicable for the completion of this document

Accessible: A site, building, facility, or portion thereof that complies with accessibility standards.

Alteration: A change to a building or facility that affects or could affect the usability of the building or facility or portion thereof. Alterations include, but are not limited to, remodeling, renovation, rehabilitation, reconstruction, historic restoration, resurfacing of circulation paths or vehicular ways, changes or rearrangement of the structural parts or elements, and changes or rearrangement in the plan configuration of walls and full-height partitions. Normal maintenance, reroofing, painting or wallpapering, or changes to mechanical and electrical systems are not alterations unless they affect the usability of the building or facility.

Asphalt: A mixture of dark bituminous pitch with sand or gravel, used for surfacing roads, flooring, roofing, etc.

Assembly Area: A building or facility, or portion thereof, used for the purpose of entertainment, educational or civic gatherings, or similar purposes. For the purposes of these requirements, assembly areas include, but are not limited to, classrooms, lecture halls, courtrooms, public meeting rooms, public hearing rooms, legislative chambers, motion picture houses, auditoria, theaters, playhouses, dinner theaters, concert halls, centers for the performing arts, amphitheaters, arenas, stadiums, grandstands, or convention centers.

Assistive Listening System (ALS): An amplification system utilizing transmitters, receivers, and coupling devices to bypass the acoustical space between a sound source and a listener by means of induction loop, radio frequency, infrared, or direct-wired equipment.

Audible Alarm: A device designed to detect of unauthorized activity which generates an audible sound on or near the premises.

Braille: A form of written language for people who are blind, in which characters are represented by patterns of raised dots that are felt with the fingertips.

Building: Any structure used or intended for supporting or sheltering any use or occupancy.

Characters: Letters, numbers, punctuation marks and typographic symbols.

Common Use: Interior or exterior circulation paths, rooms, spaces, or elements that are not for public use and are made available for the shared use of two or more people.

Cross Slope: The slope that is perpendicular to the direction of travel (see running slope). Cross slope is measured the same way as slope is measured

Curb Ramp: A curb ramp is a short ramp cutting through a curb or built up to it. If designed and constructed to be accessible, a curb ramp provides an accessible route that people with disabilities can use to safely transition from a roadway to a curbed sidewalk and vice versa.

The different parts of the most common type of curb ramp, a perpendicular curb ramp, are labeled in the illustration below. The ramp, or ramp run, is the sloped section that individuals who use wheelchairs travel up and down when transitioning between the street and the sidewalk. Transitions between the ramp and the sidewalk, gutter and street are located at the top and bottom of the ramp run. Flared sides, or flares, bring the curb itself to the level of the street. The gutter is the roadway surface immediately next to the curb ramp that runs along the curb.

Detectable Warning. A standardized surface feature built in or applied to walking surfaces or other elements to warn of hazards on a circulation path.

Dispensers: Bathroom dispensers, typically referring to soap dispensers and product dispensers, are devices that offer common bathroom products in small, single-use quantities.

Door Opening Force: The force for pushing or pulling open a door or gate.

Entrance: Any access point to a building or portion of a building or facility used for the purpose of entering. An entrance includes the approach walk, the vertical access leading to the entrance platform, the entrance platform itself, vestibule if provided, the entry door or gate, and the hardware of the entry door or gate.

Facility: All or any portion of buildings, structures, site improvements, elements, and pedestrian routes or vehicular ways located on a site.

Force: Strength or energy as an attribute of physical action or movement. Also defined as a push or pull on an object.

Grab Bars: Grab bars are safety devices designed to enable a person to maintain balance, lessen fatigue while standing, hold some of their weight while maneuvering, or have something to grab onto in case of a slip or fall.

Gravel: A loose aggregation of small water-worn or pounded stones.

Gratings: A framework of metal bars in the form of a grille set into a pavement, serving as a cover or guard to provide slip resistance in all directions.

Hall Lantern: Hall Lanterns are used to indicate the arriving elevator and the direction it will travel. In addition to the illuminated directional indicators, the hall lanterns have an audible tone to indicate direction. Audible signals sound once for elevators going up and sound twice for elevators going down.

Marked Crossing: A crosswalk or other identified path intended for pedestrian use in crossing a vehicular way.

Nosing: The horizontal, protruding edge of a stair where most foot traffic frequently occurs

Operable Part: A component of an element used to insert or withdraw objects, or to activate, deactivate, or adjust the element.

Ramp: A walking surface that has a running slope steeper than 1:20.

Running Slope: The slope that is parallel to the direction of travel (see cross slope).

Risers: Height of step in stairways.

Sales and Services: Check-out aisles, sales counters, service counters, food service lines, queues, and waiting lines.

Space: A definable area, such as a room, toilet room, hall, assembly area, entrance, storage room, alcove, courtyard, or lobby.

Story: That portion of a building or facility designed for human occupancy included between the upper surface of a floor and upper surface of the floor or roof next above. A story containing one or more mezzanines has more than one floor level.

Switchbacks: A switchback ramp design is one of the numerous configurations you can choose from. Its ramps alternate at 180-degree turns, allowing you to ascend landings and elevated spaces safely.

Tactile: An object that can be perceived using the sense of touch.

Thresholds: The threshold is the strip or wood or metal that runs across the bottom of an entry door frame.

Tread: Depth of step in stairways

TTY: An abbreviation for teletypewriter. Machinery that employs interactive text-based communication through the transmission of coded signals across the telephone network. TTYs may include, for example, devices known as TDDs (telecommunication display devices or telecommunication devices for deaf persons) or computers with special modems. TTYs are also called text telephones.

Walkway: An exterior prepared surface for pedestrian use, including pedestrian areas such as plazas and courts.

Wheelchair Space: Space for a single wheelchair and its occupant.