# Civil Rights Review – Building Accessibility

[Fill In] School District

[Fill In] High School

[ Please Fill In The Exact Date ], 2026

Facilities Packet Completed By

Name:

Title:

Date(s) of Completion:

Before signing this document, verify that the content you are signing is correct. Please sign below:

Facilities Accessibility Self-Assessment

A review of each facility where CTE classes are offered will be made. The accessibility review will include parking areas, walkways/hallways, ramps, building entrances, libraries, cafeterias, restrooms, locker rooms as well as CTE program areas/classrooms and other assembly areas as determined.

*If every building does not have an accessible restroom and/or water fountain, an accessible restroom and/or water fountain must be within near proximity, and of reasonable distance/ travel, from the non-accessible restroom and/or water fountain.

All agencies are responsible for compliance with Section 504. Applicable accessibility standards are determined by the date the facility was constructed or last renovated, as follows (ANSI, UFAS, and 2010 ADA Standards). All non-compliant areas, irrespective of built /alteration date, must be corrected in accordance with the 2010 ADA Standards.

Accessibility Standards

- Existing Facility under 504 - Built or altered beginning June 3, 1977, or earlier (34 CFR Section 104.22(a))

- American National Standards Institute (ANSI) - Built or altered between June 4, 1977, and January 17, 1991 (34 CFR Section 104.23(a))

- Uniform Federal Accessibility Standards (UFAS) - Built or altered between January 18, 1991, and January 26, 1992 (28 CFR Section 35.151.; (34 CFR Section 104.23(c)))

- 1991 Americans with Disabilities Act (ADA) - Constructed or altered on or after January 27, 1992 and before September 15, 2010 (28 CFR Section 36, Appendix D; (34 CFR Section 104.23)

- 2010 Americans with Disabilities Act (ADA) - Constructed on or after March 15, 2012 (28 CFR Section 35; (34 CFR Section 104.23)

The included checklist provides the framework for the evaluation of accessibility at the school site as described by Americans with Disabilities Act and guidelines published by the U.S. Department of Education, Office for Civil Rights.

Table of Contents

Tips for Measuring	5

Accessible Parking Spaces	8

Accessible Routes / Walkways	10

Stairways and Steps	11

Ramps	12

Curb Ramps	15

Entrances, Doors, and Gates	18

Rooms & Offices (Assembly Areas Available to Students)	20

Elevator(s) Building	22

Lifts	24

Accessible Drinking Fountains	25

Water Coolers & Bottle Fillers	27

Library	31

CTE Classroom(s)	33

Labs/Shops	35

Gymnasium/Auditorium/Weight Room/Other	37

Stadium/Field	40

Dressing, Fitting and Locker Rooms	41

Restrooms (Male/Female)	45

Public Telephones	48

Glossary of Terms	49

# Tips for Measuring:

Parking spaces:

Access aisles: Measure in inches

Parking spaces: Measure in inches

Curb Ramps: Measure in inches

Cross Slope of curb ramps: Measure in degrees or percentages or proportions

Door(s) at Entrance(s) and Exits(s):

Height of door handle: Measure Ground floor to handle in inches

Width of door(s): Measure in inches

Door opening force: The force for pushing or pulling open a door or gate; Measure in pounds

Threshold at doorway: Measure in inches

Signage:

Height if accessible signage: Measure from the floor to the centerline of sign, in inches

Height of characters of signage: Measure in inches

Elevator(s):

Height of call button: Measure from floor to center of call button panel

Width of Door: Measure in inches

Depth and Length of elevator: Measure in inches

Control height: Measure from floor to middle of call button panel, in inches

Height if Emergency Communication: Measure from floor to bottom of panel, in inches

Lift(s):

Height of controls: Measure from floor to middle of control panel, in inches

Benches: Measure from floor to the top of the bench

Toilet/Restrooms

Measure:

Height of grab bars: from floor to center line of grab bars

Height of toilet seat: from floor to toilet seat edge

Height of flush control(s): from floor to control(s)

Height of mirror: from floor to reflecting edge of mirror

Height of toilet paper dispenser: from floor to operable part of dispenser(s)

Height of soap dispenser: from floor to operable part of dispenser(s)

Height of paper towel dispenser: from floor to operable part of dispenser(s)

Height of automatic hand dryer(s): from floor to operable part of dispenser(s)

Height of disposal unit(s): from floor to top of unit(s)Height of lavatory: From floor to rim

Drinking Fountains

Measure:

Height of fountain: floor to spout outlet in inches

Height of spout: floor to tip of water spout

Height of flow: turn flow on, measure from spout to top of arc, in inches

Handrails: Height: Measure vertically from ground to top of handrail, in inches

Tables and counters:

Measure:

Tops: From floor to the table or counter top, in inches

Knee clearance (under tables, telephones), in inches:

From

- floor to lower edge of table.

- depth from table edge to leg of table

- width of work surface

Sales and Services counters require additional measurements than the reception counter tops:

- forward reach approach

- parallel approach

Curb Ramps:

Only measure the width of the ramp section of the curb ramp (labeled “ramp” on the image below). The ramp section of a curb ramp is also known as the “ramp run.” If the curb ramp has flared sides, which can also be seen in the illustration to the bottom, do not include them in the measurement. The ramp run must be at least 36 inches wide.

Running Slope:

The running slope of the curb ramp is the slope in the direction that people travel when going up or down the ramp run. The arrow in the illustration below, aligned parallel to the ramp run and perpendicular to the curb, shows where to measure the running slope.

Cross Slope:

The cross slope of the curb ramp is perpendicular to the running slope. Unlike the running slope, which runs along the ramp, the cross slope is measured across the ramp. The arrow in the illustration to the right, aligned perpendicular to the ramp run and parallel to the curb, shows where to measure the cross slope. The cross slope of a curb ramp, or any accessible route, may not exceed 2 percent.

Gutter Slope:

The gutter is the part of the street that borders the curb. To measure the gutter slope, place the level in the same position as the arrow in the illustration, with one end where the gutter meets the ramp and the other end towards the street. The gutter slope is parallel to the ramp and perpendicular to the curb.

Measurement: Slope is the measure of how steep a straight line is. For a ramp the maximum running slope allowed is 1:12. That means for every inch of height change there should be at least 12 inches of ramp run (length).

Tools Needed: Smart Level or Measuring Tape

Accessible Parking

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| SPACE COUNTS |  |  |  |  |
| Total number of parking spaces |  |  |  |  |
| Total number of accessible spaces |  |  |  |  |
| Total number of van accessible spaces |  |  |  |  |
| At least 1 of every 6 accessible spaces is van accessible? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Accessible spaces located closest to nearest accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| DIMENSIONS (INCHES) |  |  |  |  |
| Width of accessible parking space Min. 96" |  |  |  |  |
| Width of access aisle Min. 60" std ¦ Min. 96" van — ANSI/UFAS/1991 ¦ Min. 60" — 2010 if space is 132" |  |  |  |  |
| Width of van accessible space Min. 132" OR 96"+60" aisle — 2010 ¦ Min. 96"+96" — earlier |  |  |  |  |
| Width of van access aisle |  |  |  |  |
| Vertical clearance — van parking / access aisles Min. 98" |  |  |  |  |
| Ground surface slope less than 1:48? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Access aisle slope less than 1:48? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| MARKINGS & SIGNAGE |  |  |  |  |
| Accessible spaces marked with lines? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Access aisle marked to discourage parking in it? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Vertical ISA sign present? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of ISA signage (inches) |  |  |  |  |
| ISA contrasts light-on-dark or dark-on-light? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Van accessible spaces marked with lines? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| "Van Accessible" signs posted at van spaces? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of Van Accessible ISA signage (inches) |  |  |  |  |
| Individuals required to wheel/walk behind parked cars? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Detectable warnings at curb ramps in transit facilities? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Additional Information / Observations |  |  |  |  |

Accessible Routes / Walkways

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| At least one accessible route from arrival points to entrance? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Inaccessible entrances marked with directional signage? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Width of public walkways (inches) Min. 36" |  |  |  |  |
| Overhead clearance (inches) Min. 80" |  |  |  |  |
| 60" passing space at intervals along route? 2010 ADA if route is <60" wide | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Path stable, firm, slip-resistant, accessible? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Surface material (concrete / asphalt / pavers / other) |  |  |  |  |
| Walkways continuous, no steps or abrupt level changes? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Walkways blend to common level at driveways / lots? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Curb cuts at driveways and parking lots? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Protruding objects present? >4" projection between 27"–80" AFF | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If yes, describe protruding objects |  |  |  |  |
| Additional Information / Observations |  |  |  |  |

Curb Ramps

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Width of ramp run (not flared sides) — inches Min. 36" |  |  |  |  |
| Rise measurement (inches) |  |  |  |  |
| Length measurement (inches) |  |  |  |  |
| Running slope — denominator (e.g. 12 = 1:12) Max 1:12 |  |  |  |  |
| Cross slope — denominator Max 1:48 (2%) |  |  |  |  |
| Gutter slope measurement |  |  |  |  |
| Curb ramp has flared sides? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Flared sides ratio — denominator Max 1:10 |  |  |  |  |
| Landing provided at top of curb ramp? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Top landing — Length × Width (inches) Min. 36"×36" |  |  |  |  |
| Curb ramp placed diagonally at intersection? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If yes, clear space measurement (inches) |  |  |  |  |
| Raised islands in crossings? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If yes, curb ramp separation — L × W (inches) |  |  |  |  |
| Surface stable, firm, slip-resistant? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Detectable warnings provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Detectable warning dome size + spacing |  |  |  |  |
| Detectable warnings light/dark contrast? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Gratings in walking surface? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Grating space >1/2" in one direction? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Obstructions present? (describe) |  |  |  |  |
| Additional Information / Observations |  |  |  |  |

Stadium / Field

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

| Location / Building No.: |  |
| --- | --- |
| Year Built: |  |
| ADA Modification Date(s): "N/A" if none |  |
| Identify ADA Modifications: "N/A" if none |  |
| Accessible route to area? | ☐ Yes ☐ No |
| Route surface material (asphalt / concrete / gravel / dirt) |  |
| Total number of wheelchair designated seats/spaces |  |
| Wheelchair designated seating provided? | ☐ Yes ☐ No |
| Accessible route to wheelchair designated seats? | ☐ Yes ☐ No |
| Companion seats provided with wheelchair spaces? | ☐ Yes ☐ No |
| Ramp to wheelchair seating area? If yes, complete Ramps section | ☐ Yes ☐ No |
| Accessible route clear width (inches) Min. 36" |  |
| Additional Information / Observations |  |

Entrances, Doors, and Gates

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| SIGNAGE |  |  |  |  |
| Identification signage provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60" centerline; 57"–63" |  |  |  |  |
| Height of characters on signage (inches) |  |  |  |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| At double doors, signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Protruding objects within 3" of sign? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Non-glare finish on signs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Entrance marked with ISA? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| ISA contrasts light-on-dark or dark-on-light? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Inaccessible entrances have directional signage to accessible entrance? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| DOOR HARDWARE & DIMENSIONS |  |  |  |  |
| Total number of accessible entrances |  |  |  |  |
| Total number of exits |  |  |  |  |
| Ramp leading to entrance? If yes, complete Ramps section | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Width of doorway(s) (inches) Min. 32" |  |  |  |  |
| Height of door handle (inches) Max 48" |  |  |  |  |
| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If not, type of hardware on door |  |  |  |  |
| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |  |  |  |
| Threshold height — Exterior (inches) Max 1/2" |  |  |  |  |
| Threshold height — Interior (inches) Max 1/2" |  |  |  |  |
| Carpeting or mats 1/2" or less? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Floor level within 5 feet of door (direction door swings)? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Doorway platform measurement beyond each side (inches) |  |  |  |  |
| Sharp inclines and abrupt level changes avoided at threshold? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Main entrance a fire door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| ALARMS |  |  |  |  |
| Visual alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Light flashes clear or normal white? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Audible alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Stairways and Steps

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Stair location (interior / exterior) |  |  |  |  |
| Steps avoid abrupt nosing? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Stair width (inches) Min. 44" public stair |  |  |  |  |
| Handrails provided on both sides? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of handrails (inches) 30"–34" — ANSI/UFAS ¦ 34"–38" — 1991/2010 ADA |  |  |  |  |
| Length of handrails (inches) |  |  |  |  |
| Do steps have risers? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of risers (inches) Max 7" |  |  |  |  |
| Tread depth (inches) Min. 11" |  |  |  |  |
| Color/tonal contrast on stair nosings? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Ramps

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Ramp serves which area? |  |  |  |  |
| Rise measurement (inches) |  |  |  |  |
| Length measurement (inches) |  |  |  |  |
| Max rise per run (inches) Max 30" before intermediate landing |  |  |  |  |
| Running slope — denominator Max 1:12 |  |  |  |  |
| Cross slope — denominator Max 1:48 |  |  |  |  |
| Surface stable, firm, slip-resistant? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Accessible landing at top and bottom? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Top landing — L × W (inches) Min. 60"×60" |  |  |  |  |
| Bottom landing — L × W (inches) Min. 60"×60" |  |  |  |  |
| Intermediate landing between runs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Ramp changes direction? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Intermediate / direction-change landing — L × W (inches) Min. 60"×60" |  |  |  |  |
| Landing subject to wet conditions? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If yes, landing drainage slope (ratio) |  |  |  |  |
| Clear width of ramp between handrails (inches) Min. 36" |  |  |  |  |
| Handrails on both sides? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of handrails (inches) 30"–34" — ANSI/UFAS ¦ 34"–38" — 1991/2010 ADA |  |  |  |  |
| Handrails smooth + extend beyond top and bottom? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Handrails continuous? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If not continuous, length of horizontal extensions (inches) |  |  |  |  |
| Height of edge protection (inches) |  |  |  |  |
| Ramp has switchback or dogleg? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Elevators

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Building has multiple stories? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Elevator available and usable by individuals with physical disabilities? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Unassisted access to elevator? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| CALL CONTROLS |  |  |  |  |
| Height of call button — floor to panel center (inches) 15"–48" |  |  |  |  |
| Car call button size — diameter (inches) Min. 3/4" — 2010 ADA |  |  |  |  |
| Buttons easy to push or touch-sensitive? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Visible and audible signals at each hoistway entrance? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Hall lantern provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Call buttons have visible/audible signals when registered? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Audible signals: once for up, twice for down (or verbal annunciator)? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| HOISTWAY & CAR |  |  |  |  |
| Floor designations — raised characters + Braille on both door jambs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Floor designation character height (inches) Min. 2" |  |  |  |  |
| Type of entrance door (center / side / other) |  |  |  |  |
| Width of elevator entrance door (inches) Min. 32" |  |  |  |  |
| Door reopens if obstructed — remains open min. 20 seconds? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Door reopening device sensitive to light touch or contact? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Car interior — W × D (inches) Min. 51"W × 51"D ctr ¦ 68"W × 51"D side — 2010 |  |  |  |  |
| Width of car door (inches) Min. 32" |  |  |  |  |
| Emergency alarm/stop at bottom of panel? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of emergency communications — floor to bottom of panel (inches) Max 48" |  |  |  |  |
| If not accessible, accessible elevator clearly identified with ISA? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Additional Information / Observations |  |  |  |  |

Lifts

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Platform lift provides unassisted entry and exit? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Walkways have leveled platform-to-runway clearance? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Clear space for approach — L × W (inches) |  |  |  |  |
| Height of controls — floor to panel center (inches) 15"–48" |  |  |  |  |
| Low-energy, power-operated doors or gates? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Doors remain open minimum 20 seconds? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of end doors and gates (inches) Min. 32" |  |  |  |  |
| Width of side doors and gates (inches) Min. 32" |  |  |  |  |
| Lift interior — W × D (inches) Min. 32"W × 48"D |  |  |  |  |
| Additional Information / Observations |  |  |  |  |

Rooms & Offices

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Room name / type e.g. Wellness Center, Counseling Office |  |  |  |  |
| Room number(s) |  |  |  |  |
| Room/office on accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| SIGNAGE |  |  |  |  |
| Identification signage provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60" centerline; 57"–63" |  |  |  |  |
| Height of characters on signage (inches) |  |  |  |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Double doors — signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Protruding objects within 3" of sign? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Non-glare finish on signs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| DOORS |  |  |  |  |
| Number of entrances and exits |  |  |  |  |
| Fire door or exterior hinged door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of doorway(s) (inches) Min. 32" |  |  |  |  |
| Height of door handle (inches) Max 48" |  |  |  |  |
| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If not, type of hardware on door |  |  |  |  |
| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |  |  |  |
| Threshold — Exterior / Interior (inches) Max 1/2" |  |  |  |  |
| Carpeting/mats 1/2" or less? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| COUNTERS & WORKSTATIONS |  |  |  |  |
| Accessible Sales/Services reception countertop? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Forward reach approach (inches) Max 48" |  |  |  |  |
| Parallel approach (inches) |  |  |  |  |
| Accessible reception countertop? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of reception countertop (inches) Max 34" — ANSI/UFAS ¦ Max 36" — 1991/2010 |  |  |  |  |
| Countertop length × width (inches) |  |  |  |  |
| Knee clearance — H / W / D (inches) Min. 27"H / 30"W / 19"D |  |  |  |  |
| Supplies accessible by persons with disabilities? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| ALARMS |  |  |  |  |
| Visual alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Light flashes clear or normal white? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Audible alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Cafeteria

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

| Location / Building No.: |  |
| --- | --- |
| Year Built: |  |
| ADA Modification Date(s): "N/A" if none |  |
| Identify ADA Modifications: "N/A" if none |  |
| SIGNAGE |  |
| Identification signage provided? | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60"; 57"–63" |  |
| Height of characters on signage (inches) |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| Double doors — signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| Non-glare finish on signs? | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No |
| ENTRANCE & DOORS |  |
| Number of entrances and exits |  |
| Cafeteria on accessible route? | ☐ Yes ☐ No |
| Ramps leading to cafeteria? If yes, complete Ramps section | ☐ Yes ☐ No ☐ N/A |
| Width of doorway(s) (inches) Min. 32" |  |
| Height of door handle (inches) Max 48" |  |
| Door openable without grasping/twisting wrist? | ☐ Yes ☐ No |
| If not, type of hardware on door |  |
| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |
| Threshold — Exterior / Interior (inches) Max 1/2" |  |
| Carpeting/mats 1/2" or less? | ☐ Yes ☐ No ☐ N/A |
| ALARMS |  |
| Visual alarm provided? | ☐ Yes ☐ No |
| Light flashes clear or normal white? | ☐ Yes ☐ No |
| Audible alarm provided? | ☐ Yes ☐ No |
| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| INDOOR FIXED SEATING |  |
| Total number of fixed accessible seats indoors |  |
| Seats and tables on accessible route? | ☐ Yes ☐ No |
| Wheelchair seating space at fixed tables/counters? | ☐ Yes ☐ No |
| Height of fixed tables/countertops (inches) 28"–34" |  |
| Wheelchair space — W × D (inches) Min. 30"×48" |  |
| Knee clearance — H/W/D (inches) Min. 27"H / 30"W / 19"D |  |
| Accessible route between food line, seating, and exit? | ☐ Yes ☐ No |
| OUTDOOR FIXED SEATING |  |
| Total number of fixed accessible seats outdoors |  |
| Seats and tables on accessible route? | ☐ Yes ☐ No |
| Height of outdoor fixed tables/countertops (inches) 28"–34" |  |
| Outdoor surface material |  |
| FOOD SERVICE LINE(S) |  |
| Measure all food service lines including main line, à la carte, and student store lines. |  |
| Total number of food lines |  |
| Food service line aisle width (inches) Min. 36" clear |  |
| Height of food line counter / tray slide (inches) Max 34" — ANSI/UFAS ¦ Max 36" — 1991/2010 |  |
| Height of sneeze guard / food display — lowest accessible item (inches) Max 48" forward reach |  |
| Self-service items within reach range? | ☐ Yes ☐ No |
| Height of highest self-service item (inches) Max 48" |  |
| Staff-assisted alternative available if self-service not accessible? | ☐ Yes ☐ No |
| CHECKOUT COUNTER(S) |  |
| Total number of checkout aisles |  |
| Number of accessible checkout aisles Min. 1 required |  |
| Checkout aisle width (inches) Min. 36" |  |
| Height of accessible checkout counter (inches) Max 34" — ANSI/UFAS ¦ Max 36" — 1991/2010 |  |
| Width of lowered counter section (inches) Min. 36" |  |
| Knee clearance — H/W/D (inches) Min. 27"H / 30"W / 19"D |  |
| Approach type at checkout (Forward / Parallel) |  |
| Height of tableware and condiment areas (inches) Max 48" forward reach |  |
| TRAY RETURN / BEVERAGE / VENDING |  |
| Tray return station provided? | ☐ Yes ☐ No |
| Tray return on accessible route? | ☐ Yes ☐ No ☐ N/A |
| Height of tray return slot (inches) Max 48" |  |
| Clear floor space in front — W × D (inches) Min. 30"×48" |  |
| Self-service beverage station provided? | ☐ Yes ☐ No |
| Station on accessible route? | ☐ Yes ☐ No ☐ N/A |
| Height of highest operable part — beverage (inches) Max 48" |  |
| Clear floor space — beverage — W × D (inches) Min. 30"×48" |  |
| Beverage operable parts require tight grasping/pinching/twisting? | ☐ Yes ☐ No |
| Beverage force to activate exceeds 5 lbs? | ☐ Yes ☐ No |
| Vending machines provided? | ☐ Yes ☐ No |
| Vending machines on accessible route? | ☐ Yes ☐ No ☐ N/A |
| Height of vending highest operable part (inches) Max 48" forward reach |  |
| Vending clear floor space — W × D (inches) Min. 30"×48" |  |
| Vending force to activate exceeds 5 lbs? | ☐ Yes ☐ No |
| Additional Information / Observations |  |

Library

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

| Location / Building No.: |  |
| --- | --- |
| Year Built: |  |
| ADA Modification Date(s): "N/A" if none |  |
| Identify ADA Modifications: "N/A" if none |  |
| SIGNAGE |  |
| Identification signage provided? | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60"; 57"–63" |  |
| Height of characters on signage (inches) |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| Double doors — signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| Non-glare finish on signs? | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No |
| ENTRANCE & DOORS |  |
| Number of entrances and exits |  |
| Library on accessible route? | ☐ Yes ☐ No |
| Ramps leading to library? If yes, complete Ramps section | ☐ Yes ☐ No ☐ N/A |
| Overhead clearance of accessible route (inches) Min. 80" |  |
| Width of doorway(s) (inches) Min. 32" |  |
| Height of door handle (inches) Max 48" |  |
| Door openable without grasping/twisting wrist? | ☐ Yes ☐ No |
| If not, type of hardware on door |  |
| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |
| Threshold — Exterior / Interior (inches) Max 1/2" |  |
| Carpeting/mats 1/2" or less? | ☐ Yes ☐ No ☐ N/A |
| ALARMS |  |
| Visual alarm provided? | ☐ Yes ☐ No |
| Light flashes clear or normal white? | ☐ Yes ☐ No |
| Audible alarm provided? | ☐ Yes ☐ No |
| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| SEATING & WORKSTATIONS |  |
| Wheelchair seating at fixed tables/counters? | ☐ Yes ☐ No |
| Seats and tables on accessible route? | ☐ Yes ☐ No |
| Number of accessible seats |  |
| Height of fixed tables/countertops (inches) 28"–34" |  |
| Aisle space width (inches) Min. 36" ¦ Min. 60" passing — 2010 ADA |  |
| Turning space in reading areas — diameter (inches) Min. 60" |  |
| Knee clearance — H/W/D (inches) Min. 27"H / 30"W / 19"D |  |
| Accessible desk/workstations have sufficient clearance? | ☐ Yes ☐ No |
| If insufficient, alternative option available? | ☐ Yes ☐ No ☐ N/A |
| COMPUTER WORKSTATIONS & ADAPTIVE TECHNOLOGY |  |
| Computer workstations available? | ☐ Yes ☐ No |
| Accessible computer workstation(s) provided? | ☐ Yes ☐ No |
| Height of accessible workstation surface (inches) 28"–34" |  |
| Adaptive technology available? | ☐ Yes ☐ No |
| If yes, describe adaptive technology |  |
| CHECKOUT COUNTER |  |
| Number of accessible checkout areas |  |
| Height of checkout counter top (inches) Max 34" — ANSI/UFAS ¦ Max 36" — 1991/2010 |  |
| Width of lowered counter section (inches) Min. 36" |  |
| Counter top depth (inches) |  |
| Knee clearance — H/W/D (inches) Min. 27"H / 30"W / 19"D |  |
| Forward reach approach (inches) Max 48" |  |
| Parallel approach available? | ☐ Yes ☐ No |
| BOOK DROP |  |
| Measuring the Book Drop: measure from finished floor to bottom edge of drop slot, in inches. Measure clear floor space in front (W×D). Test activation force. Note approach + location. |  |
| Book drop provided? | ☐ Yes ☐ No |
| Book drop on accessible route? | ☐ Yes ☐ No ☐ N/A |
| Height of drop slot — floor to bottom edge (inches) Max 48" forward ¦ Max 54" side — pre-2010 ¦ Max 48" side — 2010 |  |
| Clear floor space in front — W × D (inches) Min. 30"×48" |  |
| Force required to activate book drop (pounds) Max 5 lbs |  |
| Operable parts require tight grasping/pinching/twisting? | ☐ Yes ☐ No |
| Approach type (Forward / Side) |  |
| Book drop location (Exterior / Interior) |  |
| CARD CATALOG & STACK AISLES |  |
| Card catalog aisle width (inches) Min. 36" |  |
| Card catalog operable part height (inches) Max 48" forward reach |  |
| Stack aisle width (inches) Min. 36" |  |
| Overhead clearance in stack aisles (inches) Min. 80" |  |
| Additional Information / Observations |  |

CTE Classroom(s)

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Room number(s) |  |  |  |  |
| Program / Course name |  |  |  |  |
| Classroom on accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| SIGNAGE |  |  |  |  |
| Identification signage provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60" centerline; 57"–63" |  |  |  |  |
| Height of characters on signage (inches) |  |  |  |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Double doors — signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Protruding objects within 3" of sign? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Non-glare finish on signs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| DOORS |  |  |  |  |
| Number of entrances and exits |  |  |  |  |
| Width of doorway(s) (inches) Min. 32" |  |  |  |  |
| Height of door handle (inches) Max 48" |  |  |  |  |
| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If not, type of hardware on door |  |  |  |  |
| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |  |  |  |
| Threshold — Exterior / Interior (inches) Max 1/2" |  |  |  |  |
| Carpeting/mats 1/2" or less? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| SEATING & WORKSTATIONS |  |  |  |  |
| Wheelchair seating at fixed tables/counters? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Number of accessible seats |  |  |  |  |
| Height of fixed tables/countertops (inches) 28"–34" |  |  |  |  |
| Aisle space width (inches) Min. 36" |  |  |  |  |
| Knee clearance — H / W / D (inches) Min. 27"H / 30"W / 19"D |  |  |  |  |
| Accessible desk/workstations have sufficient clearance? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If insufficient, alternative option available? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Supplies accessible by persons with disabilities? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| ALARMS |  |  |  |  |
| Visual alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Light flashes clear or normal white? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Audible alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Labs / Shops

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Room number(s) |  |  |  |  |
| Program / Course name |  |  |  |  |
| Lab/Shop on accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| SIGNAGE |  |  |  |  |
| Identification signage provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60" centerline; 57"–63" |  |  |  |  |
| Height of characters on signage (inches) |  |  |  |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Double doors — signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Protruding objects within 3" of sign? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Non-glare finish on signs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| DOORS |  |  |  |  |
| Number of entrances and exits |  |  |  |  |
| Width of doorway(s) (inches) Min. 32" |  |  |  |  |
| Height of door handle (inches) Max 48" |  |  |  |  |
| Door openable without grasping or twisting wrist? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| If not, type of hardware on door |  |  |  |  |
| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |  |  |  |
| Threshold — Exterior / Interior (inches) Max 1/2" |  |  |  |  |
| Carpeting/mats 1/2" or less? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| WORKSTATIONS |  |  |  |  |
| Wheelchair seating at fixed tables/counters? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of fixed tables/countertops (inches) 28"–34" |  |  |  |  |
| Aisle space width (inches) Min. 36" |  |  |  |  |
| Knee clearance — H / W / D (inches) Min. 27"H / 30"W / 19"D |  |  |  |  |
| Accessible desk/workstations have sufficient clearance? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Supplies accessible by persons with disabilities? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of soap dispenser (inches) Max 48" |  |  |  |  |
| Height of paper towel dispenser (inches) Max 48" |  |  |  |  |
| ALARMS |  |  |  |  |
| Visual alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Light flashes clear or normal white? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Audible alarm provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Gymnasium / Auditorium / Weight Room

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

| Location / Building No.: |  |
| --- | --- |
| Year Built: |  |
| ADA Modification Date(s): "N/A" if none |  |
| Identify ADA Modifications: "N/A" if none |  |
| Complete for Gymnasium, Auditorium, Weight Room, or other large assembly space. If the campus has separate Gym and Auditorium buildings, note both in Additional Information or use separate printed copies. |  |
| Space type (Gym / Aud / Weight Room / MP Room / Black Box / Other) |  |
| Room number(s) |  |
| SIGNAGE |  |
| Identification signage provided? | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60"; 57"–63" |  |
| Height of characters on signage (inches) |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No |
| Double doors — signs on nearest adjacent wall? | ☐ Yes ☐ No ☐ N/A |
| Protruding objects within 3" of sign? | ☐ Yes ☐ No |
| Non-glare finish on signs? | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No |
| ENTRANCE & DOORS |  |
| Space on accessible route? | ☐ Yes ☐ No |
| Number of entrances and exits |  |
| Width of doorway(s) (inches) Min. 32" |  |
| Height of door handle (inches) Max 48" |  |
| Door openable without grasping/twisting wrist? | ☐ Yes ☐ No |
| If not, type of hardware on door |  |
| Automatic door opener provided? | ☐ Yes ☐ No ☐ N/A |
| Door closer takes at least 3 seconds to close? | ☐ Yes ☐ No ☐ N/A |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |
| Threshold — Exterior / Interior (inches) Max 1/2" |  |
| Carpeting/mats 1/2" or less? | ☐ Yes ☐ No ☐ N/A |
| ALARMS |  |
| Visual alarm provided? | ☐ Yes ☐ No |
| Light flashes clear or normal white? | ☐ Yes ☐ No |
| Audible alarm provided? | ☐ Yes ☐ No |
| Alarm exceeds prevailing sound level in room? | ☐ Yes ☐ No |
| WHEELCHAIR SEATING |  |
| Total number of all seating |  |
| Total number of wheelchair spaces Ratio varies by capacity — 2010 ADA §221 |  |
| Wheelchair seating in different locations throughout venue? | ☐ Yes ☐ No |
| Companion seat provided with each wheelchair space? | ☐ Yes ☐ No |
| Accessible route to wheelchair designated seats? | ☐ Yes ☐ No |
| Ramp to seating area? If yes, complete Ramps section | ☐ Yes ☐ No ☐ N/A |
| Line of sight for wheelchair spaces comparable to ambulatory seating? | ☐ Yes ☐ No |
| Wheelchair space dimensions — W × D (inches) Min. 36"W × 48"D |  |
| Can supplies be accessed by persons with disabilities? | ☐ Yes ☐ No |
| ASSISTIVE LISTENING SYSTEMS |  |
| Audio amplification system provided? | ☐ Yes ☐ No |
| Adequate number of assistive listening systems (ALS) provided? | ☐ Yes ☐ No |
| Type of ALS (Induction Loop / FM / IR / Other) |  |
| Number of receivers available |  |
| If fixed audio amplification, distance from stage (ft) |  |
| Signage indicating availability of assistive listening devices? | ☐ Yes ☐ No |
| FIXED SEATING / TABLES (WEIGHT ROOM / OTHER) |  |
| Wheelchair seating at fixed tables/counters? | ☐ Yes ☐ No ☐ N/A |
| Number of accessible seats |  |
| Height of fixed tables/countertops (inches) 28"–34" |  |
| Wheelchair space — W × D (inches) Min. 30"×48" |  |
| Knee clearance — H/W/D (inches) Min. 27"H / 30"W / 19"D |  |
| Accessible desk/workstations have sufficient clearance? | ☐ Yes ☐ No ☐ N/A |
| Additional Information / Observations |  |

Locker Rooms

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Locker room type (Male / Female / Non-Gender Specific) |  |  |  |  |
| Locker room on accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| SIGNAGE & ENTRY DOOR |  |  |  |  |
| Identification signage provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of signage — floor to centerline (inches) 60" centerline; 57"–63" |  |  |  |  |
| Height of characters on signage (inches) |  |  |  |  |
| Signs mounted adjacent to latch side of door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of entrance doorway (inches) Min. 32" |  |  |  |  |
| Height of door handle (inches) Max 48" |  |  |  |  |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |  |  |  |
| INTERIOR MANEUVERING |  |  |  |  |
| Locker room has foyer? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Measurement between doors in foyer (inches) |  |  |  |  |
| Turning space — diameter (inches) Min. 60" |  |  |  |  |
| Doors swing into turning spaces? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| LOCKERS & AMENITIES |  |  |  |  |
| Number of accessible lockers ≥ 5% of each type — 2010 ADA §225 |  |  |  |  |
| Height of accessible locker operable parts (inches) Max 48" forward reach |  |  |  |  |
| Mirror provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of mirror — floor to reflecting edge (inches) Max 40" |  |  |  |  |
| Accessible benches provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of benches — floor to top (inches) 17"–19" |  |  |  |  |
| Depth of benches (inches) Min. 20"–24" |  |  |  |  |
| Back support on benches? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Clear floor space adjacent to bench (inches) Min. 30"×48" |  |  |  |  |
| SHOWER ROOMS |  |  |  |  |
| Showers in use? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Number of showers |  |  |  |  |
| Accessible shower provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Shower type (Transfer 36"×36" / Roll-in 60"×30" min) |  |  |  |  |
| Accessible shower — W × D (inches) |  |  |  |  |
| Shower has curb? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of curb in shower (inches) Max 1/2" — transfer shower |  |  |  |  |
| Shower grab bars provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Location of shower grab bars |  |  |  |  |
| Height of shower grab bars — floor to centerline (inches) 33"–36" |  |  |  |  |
| Length of shower grab bars (inches) |  |  |  |  |
| Accessible shower seat provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of shower seat — floor to top (inches) 17"–19" |  |  |  |  |
| Handheld shower/spray unit provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Handheld shower hose length (inches) Min. 59" — 2010 ADA |  |  |  |  |
| RESTROOM AMENITIES (WITHIN LOCKER ROOM) |  |  |  |  |
| Separate restroom entrance? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of restroom entrance (inches) Min. 32" |  |  |  |  |
| Turning space — diameter (inches) Min. 60" |  |  |  |  |
| Hot water available? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Hot water pipes under sink covered? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Height of sink — floor to rim (inches) Max 34" |  |  |  |  |
| Lavatory knee clearance — H/W/D (inches) Min. 27"H / 30"W / 19"D |  |  |  |  |
| Height of soap dispenser (inches) Max 48" |  |  |  |  |
| Height of mirror — floor to reflecting edge (inches) Max 40" |  |  |  |  |
| Height of paper towel dispenser (inches) Max 48" |  |  |  |  |
| ACCESSIBLE STALL (WITHIN LOCKER ROOM) |  |  |  |  |
| Designated accessible stall? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of accessible stall (inches) Min. 60" |  |  |  |  |
| Depth of accessible stall (inches) Min. 56" |  |  |  |  |
| Stall door swings out? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of stall door (inches) Min. 32" |  |  |  |  |
| Grab bars in accessible stall? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Location of grab bars (side / rear / both) |  |  |  |  |
| Height of grab bars — floor to centerline (inches) 33"–36" |  |  |  |  |
| Height of toilet seat — floor to seat edge (inches) 17"–19" |  |  |  |  |
| Height of flush controls (inches) Max 44" |  |  |  |  |
| Flush valve on transfer/open side of toilet? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of toilet paper dispenser (inches) |  |  |  |  |
| Urinal height — floor to rim (inches) Max 17" |  |  |  |  |
| Width of space around urinal (inches) Min. 30" |  |  |  |  |
| Additional Information / Observations |  |  |  |  |

Restrooms (Male / Female / All-Gender)

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Restroom type (Male / Female / Non-Gender Specific) |  |  |  |  |
| Restroom on accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Total number of accessible restrooms in building |  |  |  |  |
| If no accessible restroom in building, directional signage to nearest? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| ENTRY DOOR & SIGNAGE |  |  |  |  |
| Accessible door signage (ISA)? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of accessible signage — floor to centerline (inches) 60"; 57"–63" |  |  |  |  |
| ISA contrasts light-on-dark or dark-on-light? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Characters raised + Grade II Braille? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Signage mounted adjacent to latch side of door? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Entrance doors swing out? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of entrance doorway (inches) Min. 32" |  |  |  |  |
| Height of door handle (inches) Max 48" |  |  |  |  |
| Door opening force (pounds) Max 5 lbs — 1991/UFAS/2010 ¦ Max 8.5 — ANSI |  |  |  |  |
| Restroom has foyer? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Measurement between doors in foyer (inches) |  |  |  |  |
| INTERIOR MANEUVERING |  |  |  |  |
| Turning space — diameter (inches) Min. 60" diameter OR T-turn |  |  |  |  |
| AMENITIES |  |  |  |  |
| Hot water available? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Hot water pipes under sink covered? | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A | ☐ Yes ☐ No ☐ N/A |
| Height of sink — floor to rim (inches) Max 34" |  |  |  |  |
| Sink usable by wheelchair users? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Lavatory knee clearance — H/W/D (inches) Min. 27"H / 30"W / 19"D |  |  |  |  |
| Height of soap dispenser (inches) Max 48" |  |  |  |  |
| Height of mirror — floor to reflecting edge (inches) Max 40" |  |  |  |  |
| Height of paper towel dispenser (inches) Max 48" |  |  |  |  |
| Height of automatic hand dryer (inches) Max 48" |  |  |  |  |
| Racks, dispensers, disposal units not protruding into walkways? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| URINALS |  |  |  |  |
| Height of urinal — floor to rim (inches) Max 17" |  |  |  |  |
| Width of space around urinal (inches) Min. 30" |  |  |  |  |
| Depth of space around urinal (inches) Min. 48" |  |  |  |  |
| ACCESSIBLE STALL |  |  |  |  |
| Designated accessible stall? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of accessible stall (inches) Min. 60" |  |  |  |  |
| Depth of accessible stall (inches) Min. 56" |  |  |  |  |
| Stall door swings out? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Width of stall door (inches) Min. 32" |  |  |  |  |
| Grab bars in accessible stall? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Location of grab bars (side / rear / both) |  |  |  |  |
| Height of grab bars — floor to centerline (inches) 33"–36" |  |  |  |  |
| Length of grab bars (inches) Side ≥40" ¦ Rear ≥36" |  |  |  |  |
| Depth of grab bars from wall (inches) |  |  |  |  |
| Height of accessible toilet seat (inches) 17"–19" |  |  |  |  |
| Height of flush controls (inches) Max 44" |  |  |  |  |
| Flush valve on transfer/open side of toilet? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Obstructions to reach toilet paper dispenser? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Toilet paper dispenser operable with one hand? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Dispenser allows continuous flow of paper? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of toilet paper dispenser (inches) |  |  |  |  |
| Location of toilet tissue dispenser |  |  |  |  |
| Coat hooks/shelves provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Coat hook height — floor to hook (inches) Max 48" forward reach |  |  |  |  |
| Reach type for coat hook (forward / side) |  |  |  |  |
| Additional Information / Observations |  |  |  |  |

Accessible Drinking Fountains

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Drinking fountains provided? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Number of fountains |  |  |  |  |
| Number of accessible fountains |  |  |  |  |
| At least 50% accessible (spout ≤36")? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Fountain on accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| 30"×48" clear floor space in front? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Height of spout — floor to spout outlet (inches) Max 36" |  |  |  |  |
| Height of water flow — spout to top of arc (inches) Min. 4" |  |  |  |  |
| Water flow within 4" of spout towards front? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Up-front spout and control? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Controls mounted on front or side? |  |  |  |  |
| Controls operable with closed fist? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Operable parts require tight grasping/pinching/twisting? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Force to activate exceeds 5 lbs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Fountain protrudes into corridors or traffic ways? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Water Coolers & Bottle Fillers

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

This section records up to 4 locations side-by-side. For more locations, copy the section template from the back of this guide.

| Location / Building No.: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year Built: |  |  |  |  |
| ADA Modification Date(s): "N/A" if none |  |  |  |  |
| Identify ADA Modifications: "N/A" if none |  |  |  |  |
| Bottle filler station available? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Station on accessible route? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| 30"×48" clear floor space in front? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Station hand operated? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Station sensor operated? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Controls mounted on front or side? |  |  |  |  |
| Height of highest operable part (inches) Max 48" |  |  |  |  |
| Controls operable with closed fist? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Operable parts require tight grasping/pinching/twisting? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Force to activate exceeds 5 lbs? | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No | ☐ Yes ☐ No |
| Additional Information / Observations |  |  |  |  |

Public Telephones

Reviewer-determined standard: the Program Reviewer determines the applicable accessibility standard (ANSI A117.1, UFAS, 1991 ADA, or 2010 ADA) based on Year Built and ADA Modification Date(s) below. The LEA records measurements only.

| Location / Building No.: |  |
| --- | --- |
| Year Built: |  |
| ADA Modification Date(s): "N/A" if none |  |
| Identify ADA Modifications: "N/A" if none |  |
| Public telephone provided? If No, mark N/A and skip remaining fields | ☐ Yes ☐ No |
| Clear floor space — W × L (inches) Min. 30"×48" |  |
| Protruding objects? | ☐ Yes ☐ No |
| Push button controls? | ☐ Yes ☐ No |
| Cord length — telephone to handset (inches) Min. 29" |  |
| Approach type (Parallel / Forward) |  |
| Distance from front edge of counter to face of telephone (inches) |  |
| Highest operable part within 15"–48"? (54" max if side approach) | ☐ Yes ☐ No |
| Telephone hearing aid compatible? | ☐ Yes ☐ No |
| Volume control provided? | ☐ Yes ☐ No |
| If >4 pay telephones, TTY available? | ☐ Yes ☐ No ☐ N/A |
| Height of TTY keypad touch surface (inches) Max 48" |  |
| Telephones with TTY equipped with shelf and electrical outlet? | ☐ Yes ☐ No ☐ N/A |
| Additional Information / Observations |  |

Program Access — Facilities & M&O Staff Interview

These questions are asked of the school site's Facilities and Maintenance & Operations staff by the Program Reviewer during the on-site visit. Program Access compliance is determined OBSERVATIONALLY and through staff responses — not through physical measurement.

| Staff Member Name: |  |
| --- | --- |
| Title: |  |
| Date of Interview: |  |
| Interviewer: |  |

| ADA COORDINATION & COMPLIANCE |  |
| --- | --- |
| 1. Who is the designated ADA/Section 504 Coordinator for this facility, and how are accessibility complaints or accommodation requests handled? |  |
| 2. Has the facility undergone a formal self-evaluation under Section 504? If so, when and what were the findings? |  |
| 3. Does the facility have a current Transition Plan? When was it last updated and what items remain outstanding? |  |
| MAINTENANCE OF ACCESSIBLE FEATURES |  |
| 4. Are accessible features (ramps, lifts, elevators, accessible restrooms, automatic door openers) on the regular maintenance schedule? How frequently inspected? |  |
| 5. Are maintenance staff trained to identify and report accessibility barriers? Describe the training. |  |
| 6. Are accessible parking spaces regularly monitored to prevent unauthorized use? What is the enforcement process? |  |
| 7. How are temporary accessibility barriers (construction, equipment storage, event setup) managed to maintain accessible routes during disruptions? |  |
| PROGRAM ACCESS & RELOCATION |  |
| 8. Are there areas where a program, class, or activity has been relocated to provide program access? If so, where, why, and how was this communicated? |  |
| 9. What is the process for notifying students, parents, and staff of accessible routes, features, and accommodations? |  |
| 10. If a student with a disability needed access to an inaccessible area, what is the process for providing equivalent program access? |  |
| EMERGENCY EGRESS & SAFETY |  |
| 11. How are emergency evacuation procedures adapted for mobility impairments? Are Areas of Rescue Assistance designated, equipped, and included in drills? |  |
| 12. Are two-way communication systems at Areas of Rescue Assistance tested regularly? When last tested? |  |
| PLANNED MODIFICATIONS |  |
| 13. Are any accessibility modifications currently planned, funded, approved, or in progress? Describe scope, timeline, and funding source. |  |
| 14. Have any accessibility modifications been completed in the past five years? Describe scope and dates. |  |
| REVIEWER OBSERVATIONS |  |
| Program Reviewer observations and notes from interview |  |

# Glossary of Terms

As defined by 2010 ADA Standards

Note: The terms defined below are only applicable for the completion of this document

Accessible: A site, building, facility, or portion thereof that complies with accessibility standards.

Alteration: A change to a building or facility that affects or could affect the usability of the building or facility or portion thereof. Alterations include, but are not limited to, remodeling, renovation, rehabilitation, reconstruction, historic restoration, resurfacing of circulation paths or vehicular ways, changes or rearrangement of the structural parts or elements, and changes or rearrangement in the plan configuration of walls and full-height partitions. Normal maintenance, reroofing, painting or wallpapering, or changes to mechanical and electrical systems are not alterations unless they affect the usability of the building or facility.

Asphalt: A mixture of dark bituminous pitch with sand or gravel, used for surfacing roads, flooring, roofing, etc.

Assembly Area: A building or facility, or portion thereof, used for the purpose of entertainment, educational or civic gatherings, or similar purposes. For the purposes of these requirements, assembly areas include, but are not limited to, classrooms, lecture halls, courtrooms, public meeting rooms, public hearing rooms, legislative chambers, motion picture houses, auditoria, theaters, playhouses, dinner theaters, concert halls, centers for the performing arts, amphitheaters, arenas, stadiums, grandstands, or convention centers.

Assistive Listening System (ALS): An amplification system utilizing transmitters, receivers, and coupling devices to bypass the acoustical space between a sound source and a listener by means of induction loop, radio frequency, infrared, or direct-wired equipment.

Audible Alarm: A device designed to detect of unauthorized activity which generates an audible sound on or near the premises.

Braille: A form of written language for people who are blind, in which characters are represented by patterns of raised dots that are felt with the fingertips.

Building: Any structure used or intended for supporting or sheltering any use or occupancy.

Characters: Letters, numbers, punctuation marks and typographic symbols.

Common Use: Interior or exterior circulation paths, rooms, spaces, or elements that are not for public use and are made available for the shared use of two or more people.

Cross Slope: The slope that is perpendicular to the direction of travel (see running slope). Cross slope is measured the same way as slope is measured

Curb Ramp: A curb ramp is a short ramp cutting through a curb or built up to it. If designed and constructed to be accessible, a curb ramp provides an accessible route that people with disabilities can use to safely transition from a roadway to a curbed sidewalk and vice versa.

The different parts of the most common type of curb ramp, a perpendicular curb ramp, are labeled in the illustration below. The ramp, or ramp run, is the sloped section that individuals who use wheelchairs travel up and down when transitioning between the street and the sidewalk. Transitions between the ramp and the sidewalk, gutter and street are located at the top and bottom of the ramp run. Flared sides, or flares, bring the curb itself to the level of the street. The gutter is the roadway surface immediately next to the curb ramp that runs along the curb.

Detectable Warning. A standardized surface feature built in or applied to walking surfaces or other elements to warn of hazards on a circulation path.

Dispensers: Bathroom dispensers, typically referring to soap dispensers and product dispensers, are devices that offer common bathroom products in small, single-use quantities.

Door Opening Force: The force for pushing or pulling open a door or gate.

Entrance: Any access point to a building or portion of a building or facility used for the purpose of entering. An entrance includes the approach walk, the vertical access leading to the entrance platform, the entrance platform itself, vestibule if provided, the entry door or gate, and the hardware of the entry door or gate.

Facility: All or any portion of buildings, structures, site improvements, elements, and pedestrian routes or vehicular ways located on a site.

Force: Strength or energy as an attribute of physical action or movement. Also defined as a push or pull on an object.

Grab Bars: Grab bars are safety devices designed to enable a person to maintain balance, lessen fatigue while standing, hold some of their weight while maneuvering, or have something to grab onto in case of a slip or fall.

Gravel: A loose aggregation of small water-worn or pounded stones.

Gratings: A framework of metal bars in the form of a grille set into a pavement, serving as a cover or guard to provide slip resistance in all directions.

Hall Lantern: Hall Lanterns are used to indicate the arriving elevator and the direction it will travel. In addition to the illuminated directional indicators, the hall lanterns have an audible tone to indicate direction. Audible signals sound once for elevators going up and sound twice for elevators going down.

Marked Crossing: A crosswalk or other identified path intended for pedestrian use in crossing a vehicular way.

Nosing: The horizontal, protruding edge of a stair where most foot traffic frequently occurs

Operable Part: A component of an element used to insert or withdraw objects, or to activate, deactivate, or adjust the element.

Ramp: A walking surface that has a running slope steeper than 1:20.

Running Slope: The slope that is parallel to the direction of travel (see cross slope).

Risers: Height of step in stairways.

Sales and Services: Check-out aisles, sales counters, service counters, food service lines, queues, and waiting lines.

Space: A definable area, such as a room, toilet room, hall, assembly area, entrance, storage room, alcove, courtyard, or lobby.

Story: That portion of a building or facility designed for human occupancy included between the upper surface of a floor and upper surface of the floor or roof next above. A story containing one or more mezzanines has more than one floor level.

Switchbacks: A switchback ramp design is one of the numerous configurations you can choose from. Its ramps alternate at 180-degree turns, allowing you to ascend landings and elevated spaces safely.

Tactile: An object that can be perceived using the sense of touch.

Thresholds: The threshold is the strip or wood or metal that runs across the bottom of an entry door frame.

Tread: Depth of step in stairways

TTY: An abbreviation for teletypewriter. Machinery that employs interactive text-based communication through the transmission of coded signals across the telephone network. TTYs may include, for example, devices known as TDDs (telecommunication display devices or telecommunication devices for deaf persons) or computers with special modems. TTYs are also called text telephones.

Walkway: An exterior prepared surface for pedestrian use, including pedestrian areas such as plazas and courts.

Wheelchair Space: Space for a single wheelchair and its occupant.